S-58 5.0.0 Section 1

1. INTRODUCTION

This document was previously Appendix B1, Annex C of S-57 Edition 3.1. It specifies the checks that, at a minimum, producers of ENC validation tools should include in their validation software. This software will be used by hydrographic offices to help ensure that their ENC data are compliant with the S-57, Appendix B1 ENC Product Specification. The checklist has been compiled for the IHO from lists of checks provided by a number of hydrographic offices and software companies. The document will be maintained by means of new editions.

ENC validation software checks that the data are in conformance with the S-57 ENC Product Specification. In order to assist software developers, those checks that have been removed from all previous editions of S-58 have been retained in Edition 5.0.0 as struck out text strings. The document provides checks for individual ENC cells however additional checks applicable to ENC Exchange Sets are included in part 2.3.

1.1 Document Layout

The validation checks are laid out as follows;

Columns are as follows

1. Check number

2. Check written in defined syntax (wherever feasible) syntax defined in this document (1.2).

3. Check message to provide user with meaningful information.

4. Check response, suggested action to address warning/error.

5. Reference to UOC or PS

6. Check classification - Critical Error, Error, Warning (see 1.2)

1.2 Check Classification

The check classification is intended to ensure errors which would affect the use of the ENC in ECDIS are not included. They do not necessarily follow the strength of wording used in the S-57 ENC Product Specification or Use of the Object Catalogue for ENC. The classifications have the following meanings;

| C | Critical Error | An error which would make an ENC unusable in ECDIS through not loading or causing an ECDIS crash or presenting data upon which the user cannot navigate. |
| --- | --- | --- |
| E | Error | An error which may degrade the quality of the ENC through appearance or usability but which will not make it unusable. |
| W | Warning | An error which may be duplication or and inconsistency but which will not noticeably degrade the usability of an ENC in ECDIS. |

1.3 Guidelines on the check syntax

In order to ensure that checks can be interpreted clearly and consistently a defined syntax has been used for the reworded checks wherever possible. Each check is statement which generates a warning/error if the expression returns ‘true’.

In the below example the reworded check (in blue text) would return true and give an error for each BERTHS object which carries the attribute VERDAT;

| 1571 | Check that no BERTHS object contains the attribute VERDAT. | 4.6.2 |  |  | E |
| --- | --- | --- | --- | --- | --- |
| 1571 | For each BERTHS object where VERDAT is present. | BERTHS object includes VERDAT. | Remove values of VERACC or VERDAT. | 4.6.2 | E |

The elements of the syntax are defined as follows;

1.3.1 Operators

The following logical operators are used;

Equal

Not equal

Less than

Less than or equal to

Greater than

Greater than or equal to

AND

OR

1.3.2 Spatial Operators

Within this document operators based on those laid out in the ISO standard 19125-2 are used to describe spatial relationships tested within the checks.

They are described in annexe A of this document.

For all spatial operators a default tolerance of 0.125mm at compliation scale should be applied.

1.3.3 Values

The following terms are used for types of values;

- Present – An attribute is present either with or without a value.

- Null – An attribute has a value of null (255)

- notNull – The attribute has been populated with a value.

1.3.4 Statements

The checks must be structured using the following statements;

- If – A conditional statement which determines whether a further statement should be executed.

- For – repeat a statement until a statement is met. For the purposes of the checks the statement being met generates the error or warning specified.

- Switch – test against a variable if this does not match move on to the next test

Examples

| 1571 | Check that no BERTHS object contains the attribute VERDAT. | 4.6.2 |  |  | E |
| --- | --- | --- | --- | --- | --- |
| 1571 | For each BERTHS object where VERDAT is present. | BERTHS object includes VERDAT. | Remove values of VERACC or VERDAT. | 4.6.2 | E |

Annexe A

EQUALS – geometric object 1 is exactly equal to geometric object 2

Examples of the Equals relationship

DISJOINT – the intersection of geometric object 1 and geometric object 2 is the empty set.

(a)			       (b)				            (c)

Examples of the Disjoint relationship

TOUCHES – geometric object 1 shares one or more component (node etc) with geometric object 2.

Examples of the Touches relationship.

WITHIN –geometric 1 object is completely contained in geometric object 2

Examples of the Within relationship — Polygon/Polygon (a), Polygon/LineString (b), LineString/LineString (c), and Polygon/Point (d)

OVERLAPS -  the intersection of geometric object 1 and geometric object 2 returns a value which is not the same as geometric object 1 or 2.

CROSSES -

INTERSECTS

To be completed

CONTAINS

COINCIDENT