- S-52 Annex A

Presentation Library e4.0 draft v0.1

Date

13/05/2013 16:40:08

| Workgroup member | Line number | Clause/ Subclause | Paragraph Figure/ Table | Type of comment (General/ Technical/Editorial) | COMMENTS | Proposed change |  |  |  |  |  | OBSERVATIONS OF THE CONVENOR on each comment submitted |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 4.3 6.8 10.3.5 | 16 1 1 | Editorial | Correct symbol name in SY() instruction | Change ‘SY(CHKSYM 01) ‘ to ‘SY(CHKSYM01)’ |  |  |  |  |  | Done |
|  |  | 8.1 | 2 | Editorial | There is a wrong reference. It must be 11.6 instead of 10.6 | Change the latest sentence; For further information about location of the pivot point and the symbol definitions please see 11.6. |  |  |  |  |  | Done |
|  |  | 9 | 10 | Editorial | "Whether a parameter is optional or mandatory can be seen from the Backus-Naur diagram of the command word: " There are no Backus-Naur diagrams in the document. Optional parameters are shown in the brackets, for example: SY(SYMBOL [, ROT]); | Remove the sentence. |  |  |  |  |  | Done |
|  |  | 9.1 | STRING: | Technical | There is no information that a text string drawing on the ECDIS screen must be quoted. | Change wording of the second sentence: The STRING parameter passes a text string in single quotes that must be written on the ECDIS screen. For example: TX('DR',2,3,2,'15110',-1,1,CHBLK,50); |  |  |  |  |  | Done |
|  |  |  | SPACE: 1. | Technical | The given definition of ‘Fit’ does not explain how textual line should be fitted. Now ‘Fit’ value does not differ from 2:‘Standard’ There was the following definition in ver.3.4: “...the text string should be expanded or condensed to fit between the first and last position in a spatial object”. This explanation implies a text label shares the line geometry but it contradicts this standard requirement. That’s why this value cannot be used for ECDIS display. | Change the wording the following sentence: ‘1’ Fit – is not used (i,e. the text string should be expanded or condensed to fit between the first and last position in a spatial object) |  |  |  |  |  | Done |
|  |  |  | 3. |  | If we follow the definition of value 3: Standard (with word wrap), we must separate the label ‘Charmouth’ in the example (9.1.1) into two lines. I suppose there should be rules when and how text must be separated to lines | Add explanation to end of the sentence: “... text longer than 8 characters should be broken into separate lines by whole words.” |  |  |  |  |  | Done |
|  |  | 9.1.1 |  | Technical | Add example of the purely alphanumeric text | TX('DR',2,3,2,'15110',-1,1,CHBLK,50); |  |  |  |  |  | Needs to be done |
|  |  | 9.2.1 |  | Technical | It is useful example of a symbol rotation by the ORIENT attribute value. | Add example: The command string SY(EBBSTR01,ORIENT) selects a default ebb stream symbol rotated by the ORIENT value |  |  |  |  |  | Needs to be done |
|  |  | 9.3 |  | Editorial | There is a contradiction between sentences. In one place they say WIDTH has units that are 0.32 mm pixel diameter. However in 9.3.1. in the example they use units equalled 0.3mm. Perhaps it will be better to use pixels as units | Change the wording in the WIDTH definition: Units are 0.3 mm approximately pixel diameter |  |  |  |  |  | Done |
|  |  | 9.3 | WIDTH | Technical | There should be clearly defined. Whether or not changing the size of the dash and the interval, if the width is more 1'. How ‘WIDTH’ has an effect on the size and the interval of ‘DASH’ and ‘DOTT’ style? In my opinion ‘WIDTH’ value does not have to effect on DASH length and intervals between dashes and dots. |  |  |  |  |  |  |  |
|  |  | 9.5 | 2 | Technical | There is : “Syntax: SY(PROCNAME);” There must be CS(PROCNAME) | Change the symbology instruction abbreviation to “CS(PROCNAME)” |  |  |  |  |  | Done |
|  |  | 10.3.3 | 1 | Editorial | There is a wrong reference “(see 8.3.3.3)” There is no such clause | Change reference to 10.3.3.1 |  |  |  |  |  | Done |
|  |  | 10.3.3.8 | 3 | Editorial | There is CSP “SYMINS01” but no “NEWOBJ” | Change the wording of the latest sentence: If the SYMINS attribute is defined then it is used to symbolise the feature object using the ‘SYMINS01’ conditional symbology procedure. |  |  |  |  |  | Done |
|  |  | 10.3.4.1 | 1 | Editorial | There should be selected the instruction of the text drawing from all existing symbolization instruction because the last sentence of the paragraph points out the special priority value for Texts. | Chang wording of the first sentence: Each symbolization instruction except of text drawing from a look-up table line has a display priority given in field 4. |  |  |  |  |  | Done |
|  |  | 10.3.4.1 | 1 | Technical | Since the standard confirms (see 10.1.3), the probability overlay data from different cells, it is necessary to point out that the display priorities arranges objects from each cell separately. The exception refers to objects M_COVR with CATCOV=2 (No data). They must be drawn in the first place from all loaded cells on the screen. | Add the following end of the paragraph: Since the standard confirms (see 10.1.3), the probability overlay data from different cells, the display priorities arranges objects into each cell separately. The exception refers to objects M_COVR with CATCOV=2 (No data). Their display priority is 0 and they must be drawn in the first place from all loaded cells on the screen. |  |  |  |  |  | ???? |
|  |  | 10.3.4.2 | 2 | Technical | Since the standard confirms (see 10.1.3), the probability overlay data from different cells, it is necessary to point out that the OVERRADAR flag refers to objects from top cells | Add the following end of the paragraph: Since the standard confirms (see 10.1.3), the probability overlay data from different cells, the OVERRADAR flag refers to objects from top cells. |  |  |  |  |  | ????? |
|  |  |  |  | Technical | There are situations in Look-up table when values of priority and ‘Overradar’ layer conflict, for example: "OBSTRN","WATLEV7","SY(FLTHAZ02)","4","O","DISPLAYBASE","12410" "RESARE","","CS(RESARE03)","5","S","STANDARD","26010" Point OBSTRN has ‘overradar’ flag and the priority = 4. RESARE border is suppressed by radar but its priority is 5. What must be drawn above each other? . | Not clear. Explanations is needed. |  |  |  |  |  |  |
|  |  | 10.3.4.4 | Table | Technical | Show Correction -..." Does it refer to auto updating or manual or both ones? | Not clear. Explanations is needed. |  |  |  |  |  | Done |
|  |  | 10.3.4.6 | 2 | Editorial | There is a wrong number of Viewing groups table 13.2: “The viewing groups suggested in table 13.2...”. There must be reference 14.2 | Change the wording of the sentence: The viewing groups suggested in section 42 are intended as a framework on which the ECDIS manufacturer can base his own method of providing this capability. |  |  |  |  |  | Done |
|  |  | 10.3.4.6 | 4 | Technical | "The manufacturer may use the viewing group scheme or not..." Is it mandatory to implement or not? If it's not mandatory - then it contradicts with the phrase from the previous paragraph: "The mariner should have effective control over which features appear on the display... The viewing groups suggested in table 13.2 are intended as a framework on which the ECDIS manufacturer can base his own method of providing this capability " | Not clear. Explanations is needed. |  |  |  |  |  | Viewing group implementation has been made mandatory |
|  |  | 10.5.9 10.5.10 10.5.12 |  | General | If the Presentation library says that some objects must be selected in specific conditions, it must regulate how these objects will be drawn specific way differed from the ordinal way. There should be reference to IEC62288 (but not to IEC 61174) | Add description, how to indicate and to notify selected objects. |  |  |  |  |  | The objects listed will not change their symbology |
|  |  | 10.5.9 | 1 | Technical | The listed object must be selected in the following cases only. If they are in depth area only, i.e. they are not covered by LNDARE, FLODOC, PONTON and HULCKS objects. If we don’t allow this limitation, there will be many nonsensical selections on the ESDIC display. | Change the wording of the following sentence: The following table lists the S-57 objects and their attributes been into the depth area that satisfy this condition and must precipitate an indication within the ECDIS. |  |  |  |  |  |  |
|  |  | 10.5.9 | Table | Technical | There is no sense to list MORFAC for each CATMOR value for identical Geometric primitives. Apart from that I suppose the object MORFAC without CATMOR into the depth area is the navigational danger too. | Exchange the rows with MORFAC and CATMOR=* into one row for the class MORFAC. |  |  |  |  |  | Done |
|  |  |  |  |  |  | MORFAC |  |  |  | POINT,LINE, AREA |  |  |
|  |  | 10.5.9 | Table | Technical | Add the row LNDARE with geometric primitives: POINT, LINE. This list contains objects that are close dangerous distance from Ship Route. I suppose if own ship pass closer than the user-specified distance for point or line LNDARE, there must be selection of this objects | Add the row |  |  |  |  |  | Done |
|  |  |  |  |  |  | LNDARE |  |  |  | POINT,LINE |  |  |
|  |  | 10.5.9 | Table | Technical | The IMO standard say: “that is shallower than the mariner's safety contour” it means listed objects with bathymetric data have depth value less SAFETY_CONTOUR. Therefore the conditions for objects that have depth value have to be changed | Add the row: |  |  |  |  |  | Group Discussion |
|  |  |  |  |  |  | Marcul | VALSOU < SAFETY_CONTOUR or UNKNOWN |  |  |  | POINT,LINE, AREA |  |
|  |  |  |  |  |  | Change the rows: |  |  |  |  |  |  |
|  |  |  |  |  |  | Obstrn | VALSOU < SAFETY_CONTOUR or UNKNOWN |  |  |  | POINT,LINE, AREA |  |
|  |  |  |  |  |  | Uwtroc | VALSOU < SAFETY_CONTOUR or UNKNOWN |  |  |  | POINT |  |
|  |  |  |  |  |  | wrecks | VALSOU < SAFETY_CONTOUR or UNKNOWN |  |  |  | POINT, AREA |  |
|  |  |  |  |  |  | SOUNDG | Value of VE3D subfield < SAFETY_CONTOUR |  |  |  | 3D EDGE |  |
|  |  | 10.5.11 |  | Technical | In according with the following description of point and line LNDAREs presentation there is no any information of SAFETY_CONTOUR. Apart from that they are included in the table in 10.5.12 | Add description when and how way presentation of point and line LNDARE must be change to SAFETY_CONTOUR. Perhaps, it is the superfluous clause. Additional discussion is required |  |  |  |  |  | Group Discussion |
|  |  | 10.5.12 | 2 | Technical | IMO requirements are that the hazardous objects form a continuous safety contour. I can hardly imagine how point features in the current version of the S-52 will be involved in the formation of such a contour. Now UWTHAZXX procedure identifies to draw or not to draw a symbol “Isolated danger”, but it does not form a safety contour presentation. It is necessary to separate the two types of presentation: the safety contour and isolated danger. | Change wording in the sentence: However conditional symbology procedure UDWHAZ identifies all rocks, wrecks and obstructions that require a ISOLATED_DANGER symbol, and the output of this procedure through calling procedures OBSTRN and WRECKS must also be used in generating alarms as defined in the table above. |  |  |  |  |  | Group Discussion |
|  |  | 10.6.2 | Table, Sounding/vertical datum | Technical | What information should be present in the ECDIS Legend, if there are different values SDAT from DSPM and VERDAT from M_SDAT in the data of ECDIS display? I suppose the standard has to specify “When” manufacturer must take the value from DSPM and when from M_SDAT. The same situation with a value of vertical datum. | Add a rule to specify: when show the value sounding/vertical datum from DSPM of the cell and when - from meta objects. |  |  |  |  |  | Done |
|  |  | 10.6.2 | Table, Magnetic variation | Technical | There are many objects MAGVAR in the data. They can have different geometric primitives: point, line, area. What values should be shown in the ECDIS Legend? It should be specify in detail. | Add a rule to specify: the attribute values of what object MAGVAR should be used to form label in ECDIS Legend |  |  |  |  |  | Done |
|  |  | 10.6.3 | 6 | Technical | SIGGRP value is string value, it can include an expression with one or more pairs of round brackets. The paragraph about signal group must be change | Change wording in the paragraph: When the signal group value is set to or include “()” and/or “(1)” there is no requirement for this to be populated in the light description text. Only when the signal group value differs above mentioned values should the value be output on the ECDIS screen, this follows the paper chart convention Mariners are used to seeing. |  |  |  |  |  | Done |
|  |  |  |  |  |  | Change row in the table below: |  |  |  |  |  |  |
|  |  |  |  |  |  | Feature |  |  | LIGHTS |  |  |  |
|  |  |  |  |  |  | Light Characteristic |  |  | Flashing |  |  |  |
|  |  |  |  |  |  | Signal Group |  |  | (1) |  |  |  |
|  |  |  |  |  |  | Colour |  |  | White |  |  |  |
|  |  |  |  |  |  | Signal Period |  |  | 30 seconds |  |  |  |
|  |  |  |  |  |  | Height |  |  | 7 metres |  |  |  |
|  |  |  |  |  |  | Value of Nominal Range |  |  | 10 miles |  |  |  |
|  |  | 10.8 | Table, row #5 | Technical | They say that attribute group “C” are used for administrative information only. Actually SORDAT value affects safety navigation. Thus the value of SORDAT in the objects WRECKS, OBSTRN, UWTROC, and SOUNDG encodes the date of the reporting about underwater hazards. It says about possibility and credibility of information. The SORDAT value in DRGARE and SWPARE says about possibility changes of the depth dredged and swept area. The SORDAT value in the LNDARE, SLCONS says about date of construction or reclamation of the new coast line. | Add exception: Show values SORDAT if it is for the following objects: - WRECKS, OBSTRN, UWTROC, and SOUNDG with value QUASOU= 9 and geometry attribute QUAPOS=8 - DRGARE with QUASOU=11 - SWPARE - Any object class with attribute CONDTN=1or 3 or 5 |  |  |  |  |  | Done |
|  |  | 13.1 | 1 | General | I suppose we offer the optimization and amendment of CSP that’s way there will be changes. | Remove the following sentence: There have been no changes made to the content of the CSPs between version 3.5 and version 4.0 of the Presentation Library. |  |  |  |  |  | Done |
|  |  | 13.1.2 | 2 | Technical | Would I add the common rule for description UML. If there is a show instruction (SY(), LS(), AP() and etc), it means the graphic elements should be drawn. It is the same way as in look-up. If there is Select “<NAME>”, where <NAME> is the name of the symbol, Line or Area Pattern, it means we selected name to use in next symbology instructioin. | Add description of Actions: Graphic Instruction - to run drawing symbology on the screen in according to symbology commands, described in area 9. Select “<Name>” – to assign symboilogy name for following usage. Get <Attribute acronym> - to get attribute value from ENC object. |  |  |  |  |  |  |
|  |  | 13.1.6 | Table | Technical | There is a global parameter “Show Isolated Danger in Shallow Water” used in CSP “UWTHAZxx” | Add the rows in the table: |  |  |  |  |  |  |
|  |  |  |  |  |  | Show Isolated Danger in Shallow Water |  |  | flag indicating selection of isolated dangers symbols into shallow water area (on/off) (optional) |  |  |  |
|  |  | 13.2.1 | CLRLIN01 | Editorial | In general I agree with CARIS’s remarks | Change UML diagram see appendix file. |  |  |  |  |  |  |
|  |  | 13.2.2 | Notes: | Technical | Overscale pattern in case when only one dataset is available. Should overscale pattern be displayed in case when only one overscaled dataset is on the screen? Or just "indication" as in case of several datasets of the same compilation scale is enough? | Not clear. Explanations is needed. |  |  |  |  |  |  |
|  |  | 13.2.3 | DEPARE02 |  | Block “Initialize” means several started actions: - Get the object which is calling procedure - Get the value of the Attributes ‘DRVAL1’ and ‘DRVAL2’ | Two possible changes: - add explanation for block “Initialize” after diagram. or - remove the block “Initialize” and add blocks in diagram: “Get the Calling Object” “Get DRVAL1 and DRVAL2” See diagrams in the Appendix file. |  |  |  |  |  |  |
|  |  |  |  |  | As regards blocks “AP(DRGARE01) - and “LS(DASH,1,CHGRF)” see common description of actions. | No changes |  |  |  |  |  |  |
|  |  |  |  |  | As regards “Get RESTRN”. I suppose we need keep this block. There must be common rule. Before check attribute value must be block the getting of this attribute from an object. | No changes |  |  |  |  |  |  |
|  |  |  |  |  | As regards the lost diagram. I suppose the components diagrams were lost when they were inserted to documents. The description of diagrams elements refers to components diagrams. | See diagrams in the Appendix file. |  |  |  |  |  |  |
|  |  |  | Diagrams. DEPARE02 Continuation A | Technical | I suppose there is error in the diagram DEPARE02. Continuation A. If we answer “Yes” (true) at the question :” Is the Spatial object shared by a rivers, a lake, a canal or dock area objects?” It means there is a edge of mouth (or open water) river or lake or etc. It means there is no any line like on the Safety Contour and we need to draw it. Therefore we need to replace answers “Yes” and “No” in the diagrams | Rearrange connections “Yes” and “No” for the box ”Inland Water Objects shared?”. See the changes in the Appendix file |  |  |  |  |  |  |
|  |  |  |  | Technical | We need add the object class LOCBSN in the list of the question “Inland Water Objects shared? ” | Change the wording of the question: Is the spatial object shared by a RIVERS (river), LAKARE (lake), CANALS (canal), LOCBSN (lock basin) or DOCARE (dock area) object? |  |  |  |  |  |  |
|  |  |  |  | Technical | If there is “mouth” edge (see the row above) but it is shared by linear land area or some construction, in this case we don’t need to draw additional Safety Contour. I suppose we need to add the additional decision box to check that spatial object is shared by the following linear objects: - LNDARE (land area), GATCON (gate) or DAMCON (dam); - SLCONS (shoreline construction) or CAUSWY (causeway) with WATLEV=1,2, or 6. | Add decision box and connections with question: - “Is the Spatial Object shared at least one of the following linear objects: - LNDARE (land area), GATCON (gate) or DAMCON (dam); - SLCONS (shoreline construction) or CAUSWY (causeway) with WATLEV=1,2, or 6.” See diagrams in the Appendix file. |  |  |  |  |  |  |
|  |  | 13.2.4 | DEPCNT03 | Editorial | Change action box “Initials” to “Get the calling object” | Change name of the first action box: “Get the Calling Object”. See diagrams in the Appendix file. |  |  |  |  |  |  |
|  |  |  | Loop for each spatial object | Editorial | The diagrams was lost | Add the diagram of the loop. See diagrams in the Appendix file. |  |  |  |  |  |  |
|  |  | 13.2.5 | Description DEPVAL02 | Technical | Since DEPVAL02 is a sub-procedure, in the description should be point out the input and output parameters which transferred from / to called procedures OBSTRNnn and WRECKSnn | Add before diagram the description from initial DEPVAL02 |  |  |  |  |  |  |
|  |  |  | Diagram of the loop |  | I suppose it will be simpler if we removed decision box ‘DRVAL1?’. The check of the condition ‘DRVAL1 missing or unknown?’ includes the previous check ‘DPVAL1?’. | Remove a decision box “DRVAL1?”, see diagrams in the Appendix file. |  |  |  |  |  |  |
|  |  | 13.2.6 | LEGLIN03 diagrams | Technical | We can simplify the diagram removing the actions ‘Select Color PLRTE’ and “Select Colour APLRT”. When we say to execute symbology instructions SY(PLNSPD03) and SY(PLNSPD04), we defines that mentioned symbols will be used. However symbols description has already the colour name. I have recalled some blocks in according general rules | Remove blocks Select Colour PLRTE’ and “Select Colour APLRT. See diagrams in the Appendix file. |  |  |  |  |  |  |
|  |  |  |  | General | What does "planning/alternative routes" mean? "Monitoring/planning" as in IEC standards? | Should be clarified to avoid misunderstanding |  |  |  |  |  |  |
|  |  | 13.2.7 | LIGHTS05 Based on CARIS comments | Editorial | - Start box again -- do we need to get the one attribute when many are used and do not have an explicit get? | Fix the first block name to “Get the Calling Object”. Add new following action block with name “Get VALNMR”. See diagrams in the Appendix file. |  |  |  |  |  |  |
|  |  |  |  | Technical | CARIS wrote:- "Absent SECTR1 or SECTR2". Could this just say "Is a Sector Light?" with a note explaining what that is?” In general I agree with the comment. There should be check the light is sector or not one. I suppose we must change the condition such way that we don’t have to put in Continuation B if the calling object is not sector light. It means we need to check that there are no values of SECTR1/2 attributes. And if there are values they equal 0.0 and 360.0 correspondingly. But the block name must be “Is a NO Sector light?” We need to add explanation what mean this check. | Change the check label: - "Absent SECTR1 or SECTR2" to "Is a ‘No Sector’ Light?" Add explanation of the check after diagram: "Is a ‘No Sector’ Light?" – Are the attributes ‘SECTR1’ (sector limit one) OR ‘SECTR2’ (sector limit two) values absent OR they equal (0.00 AND 0.00) OR (0.00 AND 360.00) correspondingly? See diagrams in the Appendix file. |  |  |  |  |  |  |
|  |  |  |  | Editorial | CARIS wrote : “- Should the "Show symbol" boxes be reworded to "Draw symbol ..."?” We need to reword by common rules. | Change block names by common rule SY(…) , LS(…) and reword the explanation as “Draw…”. See diagrams in the Appendix file. |  |  |  |  |  |  |
|  |  |  |  | Editorial | CARIS wrote : “Show SY(QUESMRK1) - Show selected symbol (since we already selected the QUESMRK explicitly).” The block “Selected Symbol SY(QUESTMRK1)” is superfluous because the symbology instruction SY(QUESTMRK1) in the next block implies selection symbol name. | Remove the block “Selected Symbol SY(QUESTMRK1)” and reword next block and its note. See diagrams in the Appendix file. |  |  |  |  |  |  |
|  |  |  |  | Editorial | CARIS wrote : “Show selected symbol with ORIENT+Text Lable TE() - Lable is spelled incorrectly. In the notes for this, where do we get the direction from seaward? Could this one have a better label to add to the notes section?” We need change block name. The notes should be amended . Direction is taken from ORIENT attribute, see new block ‘Get ORIENT’ | Change block names by common rule SY(…) ,TE(…) TX(…) and reword the explanation as “Draw…” Add the action block ‘Get ORIENT’. See diagrams in the Appendix file. |  |  |  |  |  |  |
|  |  |  | LIGHTS05 Continuation B | Technical | Since we change the condition “Is a ‘No Sector’ Light?” we can remove all blocks in a logical branch for ‘No Sector’ light | Remove superfluous blocks . See diagrams in the Appendix file. |  |  |  |  |  |  |
|  |  |  |  | Editorial | I action block “SECTR2 > SECTR1? (!)” contains two operations: comparing of the sectors values and transformation of SECTR2 if it is needed. I offer to separate two blocks of decision (SECTR2 > SECTR1?) and action (SECTR2+=360) | Add new Decision block: Name:“SECTR2 > SECTR1?”; Note: Is the value ‘SECTR2’ greater than the value ‘SECTR1’? Edit the Action block “SECTR2 > SECTR1?(!)” to: Name:“SECTR2+=360” Note: Add 360 degrees to the value ‘SECTR2’ Set Control Flow arrows. |  |  |  |  |  |  |
|  |  |  |  | Editorial | We need edit blocks’ names and notes | See changes in the Appendix file. |  |  |  |  |  |  |
|  |  | 13.2.9 | OBSTRN06 diagram | Editorial | CARIS wrote: “- Starting box again (do we need to get the attribute?)” Replace the first block by two ones. | Fix the first block name to “Get the Calling Object”. Add new following action block with name “Get ‘VALSOU”. |  |  |  |  |  |  |
|  |  |  |  | Editorial | CARIS wrote:” - "Get WATLEV and EXPSOU" - these are retrieved to simply pass them into "DEPVAL02". Other procedures just have "DEPVAL02 (WATLEV and EXPSOU)" - i.e. showing the input to the procedure. Same for UDWHAZ04 - pass "DEPTH_VALUE" into it.” I suppose we don’t have to complex | Add input parameters in the block name for all sub-procedures. See changes in the Appendix file. |  |  |  |  |  |  |
|  |  |  | Continuation A | Technical | There is decision block ‘VALSOU <= 20m’ with the reference to S4 B422.5. I think it will be more correctly set definition ”dangerous” as SAFETY DEPTH set by mariner and 30m as the default value. As regards S4 B422.5 this value is provided for paper charts and it can be changed in the different HOs. | Change condition label and notes to: VALSOU <= SAFETY DEPTH? Note: Is the value of ‘VALSOU’ equal to or less SAFETY DEPTH value? See changes in the Appendix file. |  |  |  |  |  |  |
|  |  |  |  | Editorial | The block ‘Show Selected symbol’ should be renamed. Remove from the note words about sounding symbols from SNDFRM03. It must be done after the check of the local variable SOUNDING. There was no the check in the N_S diagram in the Preslib 3.4 | Update the name and the note of the block ‘Show Selected Symbols’ to: Name: SY(SELECT). Note: “Draw the selected symbol at the calling object’s location.” |  |  |  |  |  |  |
|  |  |  |  | Technical | I suppose it will be more effective if the launching of ‘SNDFRM03’ sub-procedure is moved to each Continuation A,B,C after the check of the local variable ‘SOUNDING’ | Add the block of action sub-procedure ‘SNDFRM03’ after the check of local variable ‘SOUNDING’ in the branch ‘Yes’ |  |  |  |  |  |  |
|  |  |  | Continuation B | Technical | Add the sub-procedure ‘SNDFRM03’, See comment above for ‘Continuation A’ | Add the block of action sub-procedure ‘SNDFRM03’ after the check of local variable ‘SOUNDING’ in the branch ‘Yes’ |  |  |  |  |  |  |
|  |  |  | LOOP of spatial objects | Editorial | CARIS wrote:” - This is inconsistent in the symbolize commands...” We need to change names and notes in the block where line is drawn. | Change block names by common rule LS(…) ,LC(…) and remove the explanation as “Draw…” |  |  |  |  |  |  |
|  |  |  |  | Technical | There is decision block ‘VALSOU <= 20m’ with the reference to S4 B422.5. I think it will be more correctly set definition ”dangerous” as SAFETY DEPTH set by mariner and 30m as the default value. As regards S4 B422.5 this value can be changed in the different HOs. | Change condition label and notes to: VALSOU <= SAFETY DEPTH? Note: Is the value of ‘VALSOU’ equal to or less SAFETY DEPTH value? See changes in the Appendix file. |  |  |  |  |  |  |
|  |  |  | Continuation C | Editing | CARIS wrote:” - Present area? This is being used for the area colour or pattern fill. Does this wording make sense?” It was taken from PresLib 3.4 | Reword blocks’ names by common rule AC(), LC(…), LS(…) |  |  |  |  |  |  |
|  |  |  |  |  | CARIS wrote:” Loop for Continuation C:...”. There is not a loop by spatial objects in the Continuation C. I suppose they have meant the QUALIN01 CSP, see points below. | nothing |  |  |  |  |  |  |
|  |  | 13.2.12 | QUAPOS01 | Editorial | CARIS wrote: “ - No inputs/output to ‘QUALIN’ and ’QUAPNT’ CSPs | Add input/output parameters in the description of ‘QUALIN01’ and ‘QUAPNT02’ |  |  |  |  |  |  |
|  |  |  |  |  | CARIS wrote: “ Show low accuracy symbol" - where and when? Should this have "if required added"? and a note that if this is an area, it goes in the centre?” | Add the decision block to check what QUAPNT02 returns. See changes in the Appendix file. |  |  |  |  |  |  |
|  |  | 13.2.13 | QUALIN01 | Editorial | CARIS wrote: “ Are attributes feature or spatial? - "LC(LOWACC21)" - do we draw it? Missing "Symbolize line segment" - "Of type COALNE" - what does this mean? "Is calling feature a COALNE?" would be more explicit. - "Symbolize line segment" - missing line styles: LS(SOLD,3,CHMGF);LS(SOLD,1,CSTLNE);” | Add the block ‘Get QUAPOS’ with description. Add the note ‘Symbolize line segment’ in the block of LC(LOWACC21) drawing. Change the label of decision block to ‘Is Calling Object COALNE?’ Add the drawing instructions: LS(SOLD,3,CHMGF); LS(SOLD,1,CSTLNE). See changes in the Appendix file. |  |  |  |  |  |  |
|  |  | 13.2.14 | QUAPNT02 | Editing | CARIS wrote:- " Get the calling (parent) object" - make this consistent -- is it parent or calling? | Reword block’s name, remove the word ‘(parent)’ |  |  |  |  |  |  |
|  |  |  |  | Technical | CARIS wrote:- " Do we need both the "get QUAPOS" and "QUAPOS?". This is not consistent in all of the diagrams.” | Add action block “Get QUAPOS” and decision block ‘QUAPOS?’ |  |  |  |  |  |  |
|  |  | 13.2.15 | RESARE03 | Editorial | CARIS wrote:” - Starting box again -- do we need to get the attribute values?” in our diagram it was included in one block | Separate the first action block on two ones: ‘Get the Calling Object’ and ‘Get RESTRN and CATREA’ |  |  |  |  |  |  |
|  |  |  |  | Technical | CARIS wrote:” - Some of the boxes say "Select symbol", while others are missing it….” I think the blocks ‘Symbolize area border’ and ‘Show selected symbol’ do not make sense as the central symbols and border lines can be drawn directly. | Remove blocks ‘Symbolize area border’ and ‘Show selected symbol’ Reword blocks’ names by common rule SY(…), LC(…), LS(…) Add descriptions for actions of symbolization. See changes in the Appendix file. |  |  |  |  |  |  |
|  |  |  | Continuations A, B, C, D | Technical | CARIS wrote Missing "Select" in the symbol... in the Line symbology boxes...” Yes, I agree. We need the draw instructions only. But we need move action ‘Set Priority=6’ at the beginning of the Continuation A since we need to know the priority during the drawing. I suppose, CARIS’s suggestion to move action of the border lines drawing before central symbol drawing does not make sense as in according 10.3.4.1: “ If the display priority is equal among objects, line objects have to be drawn on top of area objects whereas point objects have to be drawn on top of both.” | Move the block ‘DisplayPriority=6’ to the beginning. Remove blocks ‘Symbolize area border’ and ‘Show selected symbol’ Reword blocks’ names by common rule SY(…), LC(…), LS(…) Add descriptions for actions of symbolization. See changes in the Appendix file. |  |  |  |  |  |  |
|  |  |  | Continuation E | Technical | The same as above point except display priority. We don’t need to change priority value. | Remove blocks ‘Symbolize area border’ and ‘Show selected symbol’ Reword blocks’ names by common rule SY(…), LC(…), LS(…) Add descriptions for actions of symbolization. See changes in the Appendix file. |  |  |  |  |  |  |
|  |  | 13.2.16 | RESTRN01 | Editorial | Change action box “Get parent object” to “Get the Calling Object” | Change name of the first action box: “Get the Calling Object”. See diagrams in the Appendix file. |  |  |  |  |  |  |
|  |  | 13.2.17 | RESCSP02 | Editorial | CARIS wrote:” Missing "select symbol" in front of "default SY(...)".” | Reword blocks’ names by common rule SY(…), LC(…), LS(…) |  |  |  |  |  |  |
|  |  |  |  | Technical | CARIS wrote:” - Is there any reason to select and then draw? ” Yes, I agree. We need the draw instructions only the same way as RESARE03 CSP | Remove the block ‘Show the select symbol in the centre of the area”. See diagrams in the Appendix file. |  |  |  |  |  |  |
|  |  | 13.2.18 | SAFCON01 | Editorial | CARIS wrote:” - Box "SAFCON" - what does it mean?” | Add explanation to block name |  |  |  |  |  |  |
|  |  |  |  |  | We need to select manufacturer-optional part of the diagram | Add notes in the diagram and in the description |  |  |  |  |  |  |
|  |  |  |  |  | Blocks’ descriptions were missed. | Add blocks descriptions below diagram |  |  |  |  |  |  |
|  |  | 13.2.19 | SLCONS03 | Technical | There is a routine to check QUAPOS for point objects. I suppose we can amend this one if use reference to sub-procedure ‘QUAPNT02’ It allows to use the option of removing the low accuracy symbol from a point object | Remove blocks to check QUAPOS attribute value. Add reference to ‘QUAPNT02’ See diagrams in the Appendix file. |  |  |  |  |  |  |
|  |  |  |  | Technical | We need the draw instructions only SY(LOWACC01) if the it is selected in ‘QUAPNT02’. | Add action block SY(LOWACC01). |  |  |  |  |  |  |
|  |  |  |  | Technical | The loop for spatial component of the linear object SLCONS to draw them is the absolutely identical of the Continuation A of ‘SLCONS03’ CSP | Remove Continuation A and use the routine of drawing line objects for borders of area objects too. |  |  |  |  |  |  |
|  |  | 13.2.20 | SEABED01 description | Editorial | SHALLOW_CONTOUR is 2.0 (two) meters by default | Change the value in the bullet: “SHALLOW_CONTOUR = 2.0 [2 metres] (referred to as “SHC” in the diagrams).” |  |  |  |  |  |  |
|  |  |  | diagram | Editorial | We need edit blocks’ names and notes | Reword blocks’ names by common rule SELECT and AC(…), AP(…),See diagrams in the Appendix file. |  |  |  |  |  |  |
|  |  | 13.2.21 | SNDFRM03 description | Editorial | “Parameter(s): ... QUASOU and QUAPOS (if set)” The sub-procedure uses the value of attributes TECSOU and STATUS of the calling object. | Reword the description of : “Parameter(s): - DEPTH_VALUE passed from calling procedure; - QUAPOS of the spatial objects and QUASOU, TECSOU and STATUS of the object that are calling the main procedure. |  |  |  |  |  |  |
|  |  |  |  | Editorial | “Remarks: ... It also symbolizes swept depth and special symbols representing low reliability as indicated by attributes QUASOU and QUAPOS.” The attributes TECSOU and STATUS should be added to remarks since TECSOU defines ‘swept depth’, and STATUS defines ‘existence doubtful’ of the sounding. | Reword the following sentence in the remarks: “It also symbolizes swept depth and special symbols representing low reliability as indicated by attributes QUASOU, TECSOU, STATUS and QUAPOS” |  |  |  |  |  |  |
|  |  |  | diagram | Editorial | We need edit blocks’ names and notes | Reword blocks’ names and add notes, see diagrams in the Appendix file. |  |  |  |  |  |  |
|  |  | 13.2.22a | SOUNDG02 Description | Editorial | “Spatial Object(s): Point...” It is not quite correct. SOUNDG objects share the 3D edge spatial object , that is the array of points with three coordinates. | Reword the description of : “Spatial Object(s): 3D Edge (the array of points with three coordinates)” |  |  |  |  |  |  |
|  |  |  |  | Editorial | “Attribute(s) used: depth value from...” It is not quite correct. Depth value from 3D edge is not an attribute. It is a coordinate of a spot sounding. There are attributes QUAPOS of the spatial objects and QUASOU, TECSOU and STATUS of the object that are calling the procedure. These attributes are passed to sub-procedure ‘SNDFRM03’. | Remove the description of : “Attribute(s) used: |  |  |  |  |  |  |
|  |  |  | diagrams | Editorial | We need edit blocks’ names and notes | Reword blocks’ names by common rule SY(…) and add notes to activities blocks, see diagrams in the Appendix file. |  |  |  |  |  |  |
|  |  | 13.2.23 | TOPMAR01 diagrams | Editorial | Block “Initialize” means several started actions: - Get the object which is calling procedure - Get the value of the Attributes ‘TOPSHP’ | Remove the block “Initialize” and add blocks in diagram: “Get the Calling Object” “Get TOPSHP” See diagrams in the Appendix file. |  |  |  |  |  |  |
|  |  |  |  | Editorial | We need edit blocks’ names and notes | Reword blocks’ names by common rule “SELECT=” and SY(…) and add notes to activities blocks, |  |  |  |  |  |  |
|  |  |  | Loop’s diagram | Technical | If we use the current routine of the loop, we can get undefined value of the local variable ‘FLOATING’. It will be if there is no any platform object. Actually the diagram can be made simple if we set the first block ‘FLOATING = FALSE’ and remove this one and the checking of the rigid platform. The loop will run by all point objects in the location. However, I think, it does not increase time of the running too much. | Insert the block ‘FLOATING = FALSE’ after entry to the loop. Remove the decision block ‘Point object is of rigid class?’ and the block ‘FLOATING = FALSE’ following by the branch ‘Yes’. See diagrams in the Appendix file. |  |  |  |  |  |  |
|  |  | 13.2.24 | UDWHAZ04 description | Technical | “Spatial objects: Point, Area” We need to mention ‘Line’ spatial object since this sub-procedure run from the ‘OBSTRN06’ CSP that can be called by linear object OBSTRN. | Add ‘Line’ to the list of possible spatial object. |  |  |  |  |  |  |
|  |  |  |  | Editorial | “Attribute(s) used:...DRVAL1; DRVAL2” There is not any mention of the attribute ‘DRVAL2’. But there are checks of values of the attribute ‘WATLEV’, that is not included in the description. | Reword the description of: “Attribute(s) used: “depth range value 1” (DRVAL1); “water level effect” (WATLEV).” |  |  |  |  |  |  |
|  |  |  | Continuation A, Loop of area object | Technical | If we use the current routine of the loop, we will get Isolated Danger Symbol on DEPARE without the explicit value of DRVAL1(for instance: intertidal area). It contradicts to the initial scheme from Presentation Library 3.4. | Remove the block ‘DANGER=TRUE’ after the check ‘DRVAL1?’ by the branch ‘No’. Change the label ‘break the loop’ to ‘ ‘No’. See diagrams in the Appendix file. continue’ after the check ‘DRVAL1?’ by the branch. See diagrams in the Appendix file. |  |  |  |  |  |  |
|  |  | 13.2.27 | WRECKS04 diagrams | Technical | There are no starting action block to get the calling object and its attribute VALSOU. | Add the first block name to “Get the Calling Object” and the second action block with name “Get ‘VALSOU”. See diagrams in the Appendix file. |  |  |  |  |  |  |
|  |  |  |  | Technical | The input parameters of the sup-procedure ‘DEPVAL02’ are values of the attributes WATLEV and EXPSOU. There are wrong parameters in the name of the block that run the sub-procedure. | Change the parameters in the sub-procedure ‘DEPVAL02’ to ‘(WATLEV, EXPSOU)’ |  |  |  |  |  |  |
|  |  |  | Continuation A | Technical | There is decision block ‘VALSOU <= 20m’. I think it will be more correctly set definition ”dangerous” as SAFETY DEPTH set by mariner and 30m as the default value. As regards S4 B422.5 this value is provided for paper charts and it can be changed in the different HOs. The same conditions are ‘OBSTRN06’ | Change condition label and notes to: VALSOU <= SAFETY DEPTH? Note: Is the value of ‘VALSOU’ equal to or less SAFETY DEPTH value? See changes in the Appendix file. |  |  |  |  |  |  |
|  |  |  |  | Editorial | The blocks “Select SY(DANGER01)” and “Select SY(DANGER02)” can be changed to draw symbols directly. | Change block names by common rule ‘SY(DANGER01)’, SY(DANGER02) and add notes. See diagrams in the Appendix file. |  |  |  |  |  |  |
|  |  |  |  | Technical | There are no check: whether we draw low accuracy symbol getting from ‘QUAPNT02’ sup-procedure. | Add decision block: “did ‘QUAPNT02 return ‘LOWACC01’?”. See diagrams in the Appendix file. |  |  |  |  |  |  |
|  |  |  | Continuation B | Editorial | We need edit blocks’ names and notes | Reword blocks’ names by common rule the ‘SY(ISODRG01)’, ‘SY(LOWACC01)’ and ‘Draw Sounding Symbol(s)’. See diagrams in the Appendix file. |  |  |  |  |  |  |
|  |  |  |  | Technical | The filling colour for always dry wreck is wrong ‘CHERN’. It must be ‘CHBRN’ | Change colour names to ‘CHBRN’ wherever ‘CHERN’ is met. |  |  |  |  |  |  |
|  |  |  | Continuation B, loop for spatial objects | Editorial | The diagram of the loop for spatial objects in the Continuation ‘B’ has been lost. | Add the appropriate diagram. See diagrams in the Appendix file. |  |  |  |  |  |  |
|  |  |  |  | Technical | There is an error of block name. The line patter ‘LC( LOWACC41)’ must be instead of ‘LC( LOWACC4)’ | Change block name to: ‘Symbolize Spatial Component LC( LOWACC41)’ |  |  |  |  |  |  |
|  |  |  |  | Technical | There is decision block ‘VALSOU <= 20m’. I think it will be more correctly set definition ”dangerous” as SAFETY DEPTH set by mariner and 30m as the default value. As regards S4 B422.5 this value is provided for paper charts and it can be changed in the different HOs. The same conditions are ‘OBSTRN06’ | Change condition label and notes to: VALSOU <= SAFETY DEPTH? Note: Is the value of ‘VALSOU’ equal to or less SAFETY DEPTH value? See changes in the Appendix file. |  |  |  |  |  |  |
|  |  |  |  | Technical | I think the blocks ‘Select appropriate symbolization for the spatial component’ do not make sense as the border lines can be drawn directly by used of Look-up instructions. | Remove the block ‘Select appropriate symbolization for the spatial component’. Insert Look-up instruction as a note to the block ‘Symbolize Spatial Component’ See changes in the Appendix file. |  |  |  |  |  |  |
|  |  | 14.1 & Annex A Appendix A |  | General | Where is intersection with Annex A of IEC 62288 symbols? What symbols and colours should be used for mariner and navigational symbols? From IEC 62288, Annex A? PresLib and Chart 1 in that case are not suitable. Example: PresLib - colour of the "Clearing Line" object is NINFO (orange) IEC 62288 - red: "The line shall be drawn using a thin solid line style with the required colour red." | Not clear. Explanations is needed. |  |  |  |  |  |  |
|  |  | 14.6.2 |  | Editorial | I suppose it is better to leave the item “14.6.2 Light description Abbreviations” There should be reference to item 10.6.3. The section 14 should be one place where the user can find all abbreviations used in ECDIS display. | Restore the item “14.6.2 Light description Abbreviations” with reference to the item 10.6.3 |  |  |  |  |  | Done |
|  |  | 14.6.3 | 3 | Editorial | There are wrong references ’13.4.2 and 13.4.3. | Change the references in the sentence: “The meanings of the abbreviations in 14.6.2 and 14.6.3 must be made available to the mariner.” |  |  |  |  |  | Done |
|  |  | 16.1 | 1 | Editorial | The list of the items in the section contains wrong items numbers and names | Change the list to: 16.2 ECDIS Chart 1 16.3 Colour Differentiation Test Diagram 16.4 Plots of Symbols Arranged Alphabetically 16.5 Plot of symbols arranged alphabetically 16.6 List of symbol names and meanings arranged alphabetically |  |  |  |  |  | Done |
|  |  |  | 2 | Editorial | There are wrong references 19.4. | Change the references to 18.4. |  |  |  |  |  | Done |
|  |  | 16.2 |  | General | The images of the test diagrams must be updated in according to new Look-Up tables. | Change the images of test diagrams. |  |  |  |  |  | Needs to be updated |
|  |  | 16.6 |  | General | I suppose it will be better to keep this item because the table is easier and more convenient to use then raster image. | Restore the item “16.6” as “16.5 List of symbol names and meanings arranged alphabetically” |  |  |  |  |  |  |
|  |  | 17.8 | 1 | Editorial | There are wrong references 17.4, 19.3 and 19.4 | Change the references to 16.3,18.3 and 18.4 correspondingly |  |  |  |  |  | Done |
|  |  | Annex A | LUT tables in separate files | General | All LUT files contain text "For edition 3.5 Preslib, dated January 2014" | Change texts by "For edition 4.0 Preslib, dated January 2014" |  |  |  |  |  | Done |
|  |  | Appendix B |  | Technical | The text group for ACHARE name must be 26 instead of 21. | Change value of the text group in the symbology instruction TX() for the rows of the cases ACHARE and ACHARE with CATACH8: TX('OBJNAM',2,1,2,'15110',-1,-2,CHBLK,26) |  |  |  |  |  | Done |
|  |  | Appendix C |  |  | "ACHARE","","SY(ACHARE51);TX(OBJNAM,3,1,2,'15110',-1, 2,CHBLK,26);LC(ACHARE51);CS(RESTRN01)","3","S","STANDARD","26220" "ACHARE","CATACH8","SY(ACHARE02);TX(OBJNAM,3,1,2,'15110',-1, 2,CHBLK,26);LS(DASH,2,CHMGF);CS(RESTRN01)","3","S","STANDARD","26220" The negative XOFFS value moves left the OBJNAM label. In this case they will overlap the central symbol if HJUST=3. Text label must be move to right or change HJUST value to ‘2’ It is better to match with parameters of symbology instructions from Appendix B | Change the values of XOFFS in the symbology instruction TX() to ‘1’ instead of ‘-1’. The Look-Up rows should be next: "ACHARE","","SY(ACHARE51);TX('OBJNAM',2,1,2,'15110',-1,-2,CHBLK,26); LC(ACHARE51); CS(RESTRN01) ","3","S", "STANDARD","26220" "ACHARE","CATACH8","SY(ACHARE02); TX('OBJNAM',2,1,2,'15110',-1,-2,CHBLK,26); LS(DASH,2,CHMGF);CS(RESTRN01)","3","S","STANDARD","26220" |  |  |  |  |  | Done |
|  |  | Appendix B,C |  | Technical | When FAIRWY with a central symbol is drawn, the OBJNAM label will overlap the symbol. It happens as there are HJUST=1 and XOFFS=0. If we set HJUST=3 and XOFFS=2, the OBJNAM label will be drawn left central symbol. | Change the values of XOFFS in the symbology instruction TX() to the following way: "FAIRWY","ORIENTTRAFIC1","SY(FAIRWY51,ORIENT);TX(OBJNAM,3,2,3,'15110',2,0,CHBLK,26;LC(NAVARE51);CS(RESTRN01)","4","S","STANDARD","26050" "FAIRWY","ORIENTTRAFIC2","SY(FAIRWY51,ORIENT);TX(OBJNAM,3,2,3,'15110',2,0,CHBLK,26);LC(NAVARE51);CS(RESTRN01)","4","S","STANDARD","26050" I-103 "FAIRWY","ORIENTTRAFIC3","SY(FAIRWY51,ORIENT);TX(OBJNAM,3,2,3,'15110',2,0,CHBLK,26;LC(NAVARE51);CS(RESTRN01)","4","S","STANDARD","26050" "FAIRWY","ORIENTTRAFIC4","SY(FAIRWY52,ORIENT);TX(OBJNAM,3,2,3,'15110',2,0,CHBLK,26;LC(NAVARE51);CS(RESTRN01)","4","S","STANDARD","26050" |  |  |  |  |  | Done |
|  |  | Appendix D |  | Technical | "SAFCON","","LS(SOLD,3,DEPSC)","8","O","DISPLAYBASE","13000" We cannot use this instruction in Look-up table, because the 1st field is ‘code of the object class’. There is no object class “SAFCON” in S-57 edition 3.1. They mention about “safcon” as named presentation, i.e. it is “safcon” = “LS(SOLD,3,DEPSC)","8","O","DISPLAYBASE","13000" | Remove the row from the Look-up table. Add description of presentation named ‘safcon’ in item 10.5.11 |  |  |  |  |  | Done |
|  |  | Appendix E,F |  | Technical | It is better, to change the rows of VEGATN objects presentations such way to draw common case: "VEGATN","","","","","","". Actually there are a lot of cases when visual conspicuous tree encoded with CATVEG=<unknown>. It is important observation object but used of the current Look-up table it will not be drawn. I suggest the following changes: | Change all rows for VEGATN object to the following: "VEGATN","","SY(TREPNT04)","3","S","OTHER","32030" "VEGATN","CATVEG7","SY(TREPNT05)","3","S","OTHER","32030" "VEGATN","CATVEG21","SY(TREPNT05)","3","S","OTHER","32030" |  |  |  |  |  | Done |