S-52 Annex A

Presentation Library e4.0 draft v0.1

Date

17th May 2013

| Workgroup member | Line number | Clause/ Subclause | Paragraph Figure/ Table | Type of comment (General/ Technical/Editorial) | COMMENTS | Proposed change | OBSERVATIONS OF THE CONVENOR on each comment submitted |
| --- | --- | --- | --- | --- | --- | --- | --- |
| HP-01 |  |  |  | General | Too many changes - Impossible to review thoroughly all changed details The S-52 contains a huge number of changes, most of which are not intended to change anything, but to make it looking different. The original task of review was hopeless until the redlined version was provided. However the task of review of 475 pages is so huge that we cannot assume that deep enough expert review has been executed to the document in order to avoid unwanted mistakes from the change of style without intention to change the requirement. Further after the publication of the new edition the readers of it will have huge task to compare what is changed and what is not changed. This situation is made even worse by recognizing that the “cover letter” document “S-52 DIPWG Paper_Documented Changes.doc” submitted to assist the review do not list all intended changes. For example 10.9.1, 10.9.2 are not included into this document. Especially some items declared as “clarifications” contain huge number of changes. Note that for example the “clarification” change of chapter 14 contains 162 pages rewritten rules !!! As a conclusion we are very worried about the quality of the new edition. It was a mature document which required a limited number of changes and clarifications. Now it may contain any number of not found mistakes. |  |  |
| HP-05 |  | 10.1.2 |  | General | About unknown and missing attributes This issue is severe. It seems that in practice so far IHO has not been able to force member states to follow IHO standards. It is known that IHO has an attempt to improve the situation by publishing a new edition of IHO S-58 Checks for ENC charts. This new edition is assumed to form a legal basis to declare that an ENC charts which do not pass S-58 is not a legal version to be published as an ENC to be used by an ECDIS mandated by IMO. The drafted change for 10.1.2 assume that even after new edition of S-58 the situation of non-legal ENC charts will continue. If this is the case then every look-up table entry and every CSP shall be amended to define what to do in case the mandatory attribute is missing and what to do when mandatory attribute has unknown value. It does not help anybody to say that the IHO is powerless in this case, if what to do for presentation is left open for multiple interpretations. | If the principle of IHO member states slipping from the IHO rules is accepted, then IHO shall review and, if required amend, each look-up table entry and each CSP to be fail-to-safe for two separate cases of missing or unknown mandatory attribute. |  |
| HP-06 |  | 10.1.3 |  | Technical | About Cell overlap Background: IHO inability to control member states and force them to follow what is specified in the S-57 Current proposal will create a new ECDIS anomaly. Reason is that current proposal will make users to expect that OEM will handle the difficult situation when IHO member state(s) has produces overlapping cells, but the proposed text does not give any instruction how to do that. The result will be as many styles as there are different OEMs. The serious overlapping problem is not only related to drawing but also related to chart based alerts and indications, which should be based on the largest scale available. Mixed drawing of the skin of the earth objects is very difficult. Reason is that they cannot be drawn as an overlay to each other. Mixed drawing of other objects than the skin of the earth is possible, but will lead to end user confusion with double contour, double soundings, double aids to navigation, etc. IHO DIPWG shall agree what to do in this case. Technical alternatives include at least - Fixed drawing priority based on a criteria, for example alphabetical order of name of the producer. This priority used also for alerts and indication. - Mixed drawing of non shin of the earth objects. Fixed priority of drawing of skin of the earth objects based on a criteria, for example alphabetical order of name of the producer. This alternative is against current drawing methods and will cause large software changes. Also it may be too difficult for some existing ECDIS to implement. - Propose your own better idea | Force IHO member states to follow what is specified in the S-57 If this is done the complete chapter about data overlap can be removed. If above not possible to accept, then IHO DIPWG must define exact drawing rules plus alerts and indications rule for overlapping cells. |  |
| HP-10 |  | 10.5.9 |  | Technical | About detection and indication of navigational hazards We agree basic principle of this change. Our opinion is that the method of indication shall be defined to avoid an open door for multiple different implementation by different OEMs. This feature shall not be subject to variations and shall be identical in every ECDIS. | Change as below The IMO Performance Standard for ECDIS requires that "ECDIS should give an indication, if, within a specified time set by the mariner, own ship pass closer than the user-specified distance for a danger (e.g. obstruction, wreck, rock) that is shallower than the mariner's safety contour or an aid to navigation”. The following table lists the S-57 objects and their attributes that satisfy this condition and must precipitate an indication within the ECDIS. The point, line or area shall be graphically indicated using the presentation named as “dnghlt” in the look-up tables. | Done |
| HP-20 |  | 14 | Whole chapter | General / Technical | About rewritten CSPs All technical descriptions are made in a new style. The version submitted for review seems to contain a huge number of already “not passing the review” indicating comments. Some picks from the document: - My opinion, it is better to remove …. - There was the following action: “show the selected symbol …..”. It should be changed ….” All above indicate that the rewritten chapter 14 is far away from being acceptable for publishing. We wonder how this kind of large task could be completed with a safe and sound result. The risk of major disaster after publishing of the new edition of the S-52 is very high. | Consider keeping this part unchanged. Consider only minor added notes to clarify the content of the previous edition 3.4. Consider keeping the “portrayal for S-100/S-101 project” as a separate task and not included in the S-52 ed 3.5 or 4.0. This would allow enough time and resources to a have both S-52 and S-100/S-101 as sound and solid standards. |  |

| HP-23 |  | 2.21 |  | Technical | About proportional size of symbols The symbol size shall be independent of the size of the display. This is essential to benefit from larger and therefore more expensive display. The larger size should be used to enlarge geographical coverage. The proportional increase serves only longer viewing distance, but does not serve larger geographical coverage. | Change as below: The size is specified in normalized units of 0.01 mm. The minimum dimension is always more than 4 mm. This size applies to display on a standard minimum screen specified in PC&SS [5] and S-52 [3]. The size should be enlarged in proportion to increase in side length for larger screens | Done |
| --- | --- | --- | --- | --- | --- | --- | --- |
| HP-24 |  | 7 |  | Technical | About User Interface colors Originally in the '90s there were no presentation rules for the navigation instruments by the IMO. Therefore it was natural that the IHO included also mandated user interface colors. In 2004 IMO published MSC.191(79) Performance Standards for the Presentation of Navigation-related Information on Shipborne Navigational Displays. IEC has published related testing standard IEC 62288. Today MSC.191(79) and IEC 62288 set rules for user interfaces. Therefore IHO should remove from the S-52 all rules for the user interface colors. IHO should concentrate on colors for charted objects and colors for the information overlaid on top of the charted information. | Delete as below: Because user interfaces based on window systems will have a strong influence on the design of ECDIS Systems, colours for the user interface are included in the colour scheme as well. | Removed User Interface colours |
| HP-25 |  | 10.3.4.4 |  | Technical | About Independen mariner selection The chapter for “Independent mariner selections” is not completed. Difficult to comment the current version. Main comment is that the topic of the chapter is important for avoiding ECDIS anomalies. A new review shall be organized when the draft is completed. Also very important is to add a test for each function into the S-64. |  |  |
| HP-26 |  | 10.3.4.6 |  | Technical | Small inconsistencies as result of changes in other chapters Fix small mistakes in numbers of the referenced chapters. Make requirement consistent: Minimum implementation is described in 14.3 and 14.5 while 14.2 and 14.4 give further possibilities for a manufacturer to provide more display “on” or “off” switches. Make terminology consistent with 14.3 – 14.5: | Change as below: The mariner should have effective control over which features appear on the display (subject to the over-riding requirements of IMO category), as required by the IMO ECDIS Performance Standard section 3.5. The viewing groups suggested in tables 134.2 and 14.4 are intended as a framework on which the ECDIS manufacturer can base his own method of providing this capability. The minimum mandatory implementation is described in 14.3 and 14.5. Viewing groups are 'on' or 'off' switches for use by the mariner to control the information appearing on the display. An item in the viewing group table may be a chart object; a mariners' or other time-variable object; a special symbol such as the "depth less than safety contour" pattern; or a non-ENC feature such as the shallow water pattern. In edition 3.3 further 'symbol viewing groups' have been added, to allow auxiliary symbols such as contour labels, the 'low accuracy' symbol, etc., to be switched on or off without affecting the primary symbolisation of the object. Items in the viewing group tables in section 154.2 are arranged in numbered groups (e.g. group 26230 consisting of the items pipeline area and cable area) which in turn are arranged in setslayers (e.g. setlayer 26000 consisting of cautionary areas). The groups are arranged by IMO Category, in the sequence of INT 1 [1]. The manufacturer may use the viewing group scheme or not, as he prefers. If he does use it, then in some cases a single item, such as soundings (33010) should be selectable. In other cases several groups from different sets may be combined. | Done |
| HP-27 |  | 10.4.2 |  | Technical | About turning off SCAMIN The new requirement “ECDIS must provide a function to turn off the SCAMIN attribute to display all objects in the chart display” requires corresponding test in S-64 |  | OK |
| HP-28 |  | 10.5.9 |  |  | About Alarm or Alert IMO has used alert since MSC.252(83):2007 |  | OK |