S-52 Annex A

Presentation Library e4.0 draft v0.1

Date

May 2013

PLB = Pol Le Bihan (Geomod/Dcns)

| Workgroup member | Line number | Clause/ Subclause | Paragraph Figure/ Table | Type of comment (General/ Technical/Editorial) | COMMENTS | Proposed change | OBSERVATIONS OF THE CONVENOR on each comment submitted |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PLB |  |  |  | General | This review is a good point because it forces us to clarify the PL but we need time to realize a complete review in details, make proposals, and test this new edition. By doing this new review we will add clarifications but also we take the risk to introduce new confusing points (eg transferring Data Coverage procedure to UML is confusing to my point of view) |  |  |
| PLB |  |  |  | General | An important point is to take the opportunity of this new edition to clearly separate anti-grounding procedures for rendering (display) to the anti-grounding procedures for alerts. This means, we need to define new specific procedures for alerts. The first aim of the existing procedures UDWHAZnn and DEPAREnn is rendering. Using them in the definition “(detecting crossing safety contour)” lead to confusing points. |  |  |
| PLB |  |  |  | General | I use this opportunity to make other proposals which the objective is to make the situation clearer for the mariner. The row cells for these proposals are in this colour |  |  |
| Review in details |  |  |  |  |  |  |  |
| PLB |  | Basic Concept of ECDIS display | 5 | Editorial | The point 2 make a reference of a “display level” What is the definition of a display level ? | Define the term “Display Level” | Display Category done! |
| PLB |  | Symbology Instructions | 6 | Technical | Logical look up table entry allows to call for more than one conditional procedure | Add ‘s’ to procedures Change also the 10.3.3.5 UML Diagram, How to use the Lookup Tables | Done - |
| PLB |  | User Interface Colours | 7.1.7 | Editorial | Contradiction between the note “These colours are to be used whenever a user interface is on the same screen as the chart display.“ and the begin of the paragraph “Use of these specific colours for user interfaces is optional.” | Review the note or the text | Removed whole section on User Interface Colours |
|  |  | Unknown or Missing Mandatory Attributes | 10.1.2 | Editorial | .. producer may encode the attribute as unknown. | … Producer may encode the attribute value as unknown. | Done |
| PLB |  | Overlaps | 10.1.3 | Technical | “To enable the ECDIS to still function correctly OEMs must build contingencies into their systems to cope with overlaps in disputed areas”. | Need more accurate technical specifications What means build contingencies? | ???? |
| PLB |  | Gaps | 10.1.4 | Technical | “During route monitoring the Mariners attention must be drawn to areas where there are gaps in the data coverage” | Need more accurate technical specifications What means build contingencies? | ???? |
| PLB |  | How to use the Look-Up Tables | 10.3 | Editorial | The word look up table “line” replace the word look up table “entry” | Entry is more appropriate Than Line | Done |
| PLB |  | Symbolizing a non-ENC object class | 10.3.3.4 | Editorial | “... which on cursor inquiry of attributes INFORM and/or TXTDSC would display text explaining the object in the SENC …” It is not sure that INFORM and/or TXTDSC are encoded to explain the object | The ECDIS presentation should take the symbol ('QUESMRK1') should be shown as a fail-safe presentation, which on cursor inquiry of attributes INFORM and or TXTDSC would display text explaining the object in the SENC. All standard S-57 attributes permitted for ENCs that have been populated, must be available for cursor enquiry (Inform or TxtDsc for example could explain the object in the SENC) | No |
| PLB |  | Undefined atttributes | 10.3.3.6 | Editorial | … If these attributes are not undefined …. | Replace by If these attributes are undefined | Done |
| PLB |  | How to use the look up tables | 10.3.3.5 | Technical | The end of diagram UML from “Is the selected symbology instruction … “ needs clarification | Proposal - Initiate an empty list of symbology instructions. - Loop on each symbology instructions from the selected look up table entry. - Is the current symbology instructions is a Conditional Symbology procedure. - Yes, Process conditional symbology procedure. Complete the initial list with the resulting symbology instructions. - No, add the current symbology instructions to the initial list. - Applies the list of symbology instructions to symbolize the object | EM |
| PLB |  | Undefined atttributes | 10.3.3.6 | Editorial | CS(RESTRnn) is not a good example here => RESTRNnn is not an attribute acronym parameter | Remove Cs(RESTRNnn) of the text | Done |
| PLB |  | No symbol objects | 10.3.3.7 | Technical | about “The empty entry into the look up table is simply for completeness” “ | I propose to note that “no symbol objects” must be available for the cursor enquiry service. In consequence the entry is not empty and is filling with the adequate display category and viewing group. (reported in the look up table document) Ref TSMAD22/DIPWG3 08.5B | Meta objects given display category and viewing group. ◦ "M_ACCY","","","","","OTHER","31010" ◦ "M_HOPA","","","","","OTHER","31010" ◦ "M_SDAT","","","","","OTHER","31010" ◦ "M_SREL","","","","","OTHER","31010" ◦ "M_VDAT","","","","","OTHER","31010" |
| PLB |  | Display Priority | 10.3.4.1 | Technical | Nothing about lines with same display priorities | Two coincident lines with the same display priorities must be symbolized both. | ??? |
| PLB |  | Display Priority | 10.3.4.1 | Technical | No attention is made about the fact that drawing engine must execute all symbol instructions of a display priority “p”, that belong to the same scale range before to execute symbol instructions of the display priority “p + 1”. Note : symbol instructions of a scale range mean : Groups all symbol instructions with the same display priority for all objects of SENCs that have the same intended usage) | Add this notion | Do we need to specify |
| PLB |  | Display Priority | 10.3.4.1 | Technical | The opportunity to change some display priorities in order to avoid that some point objects could be hidden by fill colour instructions from area objects. | Change display priorities as propose in the paper “display priorities” (reported in the look up table document) Ref TSMAD22/DIPWG3 08.5B |  |
| PLB |  | Independent Mariner Selections | 10.3.4.4 | Editorial | M-quality Objects National Names | Need to make a reference to a paragraph : M-quality Objec == Ecdis Legend ?? National Names == National Attributs??? |  |
| PLB |  | Date Dependent Objects | 10.4.1 | Technical | The possibility to display objects for any required date could be useful. I don’t support the fact to remove this possibility | Keep the functionality (or the possibility) | This was never an option |
| PLB |  | Date Dependent Objects | 10.4.1 | Technical | Proposal to add the possibility to distinguish date dependant objects by a specific symbol | Use the same principle as information objects but with a D at the place of the I | Good Idea |
| PLB |  | Detection and Notification of Navigational Hazards | 10.5.9 | Technical | UDWHAZ04 “DANGER=False” Is dependant of the user parameter “show isolated danger in shallow water” ? | By this Do we want really offer the possibility to raise an alert - On all no dangerous wrecks obstructions? - Or on dangerous wrecks obstructions (shallower that the safety depth) in unsafe waters |  |
| PLB |  | Detection and Notification of Navigational Hazards | 10.5.9 | General | The possibility to raise alert on Shoreline, bridge pylons, no dangerous wrecks, . could lead to huge number of additional and useless alerts (taking account the fact that an alert will be raised on safety contour anyway. | Confirm that we need raise alert on no safety contour limits as land area shoreline construction |  |
| PLB |  | Detection of Areas, for which Special Conditions Exist. | 10.5.10 | Technical | - Areas with restrictions are not limited to restricted areas. All the following objects support the attribute restriction (ACHARE,CBLARE,DWRTPT,DRGARE,DMPGRD,FAIRWY,ICNARE,ISTZNE,MARCUL,MIPARE,OSPARE,PIPARE,PRCARE,RESARE,SPLARE,SUBTLN,TESARE,TSSCRS,TSSLPT,TSSRON,NEWOBJ,) | Add a generic filter “any feature that support the attribute restriction” | Offers the possibility to distinguish alerts from “entry restriction” “anchoring restriction” “others restrictions” |
| PLB |  | Detection of Areas, for which Special Conditions Exist. | 10.5.10 | Technical | Areas to be avoided RESARE RESTRN = 14 - Areas to be avoided could be encoded also with CATREA = 10 (The territorial sea area that must be raised as an alert in new test for ECDIS, is encoded as TESARE CATREA 10 ) | Add a filter for any feature type with RESTRN = 14 or CATREA = 10 |  |
| PLB |  | Detection of Safety Contour | 10.5.12 | Technical | Need clarification if area objects as flotting doc , pontoon , land area must raise an alert even if they are not in limit of unsafe/safe waters ? | Clarification is needed to distinguish between the safety contour limit that is drawing (DEPARE02) and the safety contour limit for alerts (both are not always the same) | Proposal to define conditional procedure for alert anti-grounding separately from condition procedure for rendering anti-grounding. |
| PLB |  | Detection of Safety Contour | 10.5.12 | Technical | UDWHAZ04=true | Clarifications is needed to distinguish between to draw a danger with isolated danger symbol (UDWHAZ04=true) and to raise an alarm on an danger (some of them are not drawn with isolated danger symbol (UDWHAZ04=false) |  |
| PLB |  | Identifying Automatic Chart Corrections On Mariners Demand | 10.7.1.5 | Technical | How to distinguish automatic updates ? free ? Is distinguish deleted objects needed ? => important to constitute the SENC. |  |  |
| PLB |  | Sorting by Significance | 10.8.4 | Technical | Sorting by display priority | Proposal sorting by display priorities and geometric delineation when priority is equal . (sorted by point, line area) |  |
| PLB |  | Cursor Pick | 10.8 | Editorial | I don’t support to remove “The chapter “Spatial and meta-objects, collection objects” | Keep spatial and collection objects specifications And add specifications on “no symbol objects” |  |
| PLB |  | Look-Up Table Listings |  | Technical | See proposal to complete “look up tables” in accordance with remarks on display priorities and categories above. |  |  |
| PLB |  | Conditional Symbology Procedure 'DATCVR02' | 13.2.02 | Proposal | Take the opportunity of this review to clarify “OverScale Pattern rendering stragtegy” The logic to render or not Overscale pattern is confusing for the mariner | Proposal - If the display scale is larger than twice of the cell compilation scale then the overscale pattern must appear on the visible part of the cell (in any case and for any image composition) - If the ship’s position is over a display cell that is in overscale situation then an overscale indication must be raised. |  |
| PLB |  | Conditional Symbology Procedure 'DATCVR02' | 13.2.02 | Technical | Chart Scale range boundaries when the navigation purpose of the data changes should be symbolised on the ECDIS display a simple linestyle | Proposal Complex line style SCLBDYnn renders Chart scale range boundaries more prominent and less confusing than simple line Proposal : The best presentation LC(SCLBDYnn) is the first choice and the alternative is simple linestyle |  |
| PLB |  | DEPARE02 |  | Editorial | Not complete ? |  |  |
| PLB |  | UDWHAZ04 |  | Technical | - The name of the boolean variable DANGER is confusing - Two loops on group one objects are described to detect the underlying depth areas are not efficient. Simplify by using one loop. - The question “Show isolated danger in unsafe areas” is not clearly expressed in the diagram ? | Proposal The Boolean variable DANGER (true, false) is replace by an enumerated variable DANGER_LOCATION with - SafeWatersLocation - UnsafeWatersLocation - IntertidalLocation Realized only one loop for detecting the underlying depth areas |  |