S-52 Annex A

Presentation Library e4.0 draft v0.1

Date

28th Mar 2013

| Workgroup member | Line number | Clause/ Subclause | Paragraph Figure/ Table | Type of comment (General/ Technical/Editorial) | COMMENTS | Proposed change | OBSERVATIONS OF THE CONVENOR on each comment submitted |
| --- | --- | --- | --- | --- | --- | --- | --- |
| HP-01 |  |  |  | General | Too many changes - Impossible to review thoroughly all changed details The S-52 contains a huge number of changes, most of which are not intended to change anything, but to make it looking different. The original task of review was hopeless until the redlined version was provided. However the task of review of 475 pages is so huge that we cannot assume that deep enough expert review has been executed to the document in order to avoid unwanted mistakes from the change of style without intention to change the requirement. Further after the publication of the new edition the readers of it will have huge task to compare what is changed and what is not changed. This situation is made even worse by recognizing that the “cover letter” document “S-52 DIPWG Paper_Documented Changes.doc” submitted to assist the review do not list all intended changes. For example 10.9.1, 10.9.2 are not included into this document. Especially some items declared as “clarifications” contain huge number of changes. Note that for example the “clarification” change of chapter 14 contains 162 pages rewritten rules !!! As a conclusion we are very worried about the quality of the new edition. It was a mature document which required a limited number of changes and clarifications. Now it may contain any number of not found mistakes. |  |  |
| HP-02 |  | 8.4.2 |  | Technical | About transparent fill The presentation library has always allowed two different technical methods to implement transparent fill. Although the second method is more difficult to implement it is as legal as the first method. We have created several generation of ECDIS models since nineties and we have experience from both methods in the end user product. The drafted change is confusing. It starts by saying that the presentation library supports two methods, but then it says that only first method is used. This new wording is not acceptable. | Change as below The presentation library supports two methods of transparent fill. 1.) with only a percentage of the pixels having the fill colour (stippled fill, pseudo transparency); 2.) by mixing the fill and underlying colour at each pixel, according to the fill percentage. It is very likely that most of the ECDIS systems do a transparent fill with method 1 and 4 pixel groups. In this case the method 1 can only implement the 25%, 50% and 75% percentages for the transparency. For compatibility with both transparency methods only percentage values 25%, 50% and 75% are used within the presentation library. Only method 1 is used by the Presentation Library. Description of method 1 with 4 pixel groups. That means e.g. if If an area of 4 by 4 pixels has to be filled with a transparent colour only 3, 2 or 1 pixel(s) of this area are tinted with the opaque fill colour while the remaining pixel(s) are tinted using the colour 'TRNSP' (= 100% transparent, see 4.2.1), which means the colour fill is not performed for these pixels. Thus the colour of the underlying pixels still can be seen through. On a high resolution screen the result will be very close to a real transparent fill. The following explains the pseudo-transparency that can be achieved by this method: <pictures to explain pseudo transparency> Because it is very likely that most of the ECDIS systems do a transparent fill with this technique only the 25%, 50% and 75% percentages for the transparency are used within the presentation library. |  |
| HP-03 |  | 8.5.3 |  | Technical / Editorial | About calculating the centre of gravity This issue is technically extremely difficult and together with the current text it will lead to new anomalies (anomaly = ECDIS operates in different way than expected and there is a great variability between OEMs) The target of the requirement can be supported, but the target must be very carefully secured by giving exact examples when and where to display a centred symbol. “when” is part of this issue as the rule in 8.5.1 specify that the centred symbol shall not be drawn if it is not within the corresponding area when drawn including the offset from the centre of gravity. | Change as below A method for finding the centre of and gravity of an area pattern must be adopted by the ECDIS manufacturer. This must ensure that the correct symbol is viewable on screen even when the area is only partially on screen. It must also be robust enough to uniquely identify areas which are concave, and where the centre is outside the area (for example, in a banana bananna shaped area the centre is not within the area but the symbol must be drawn inside.). It shall be noted that drawing of the centred symbol requires that the complete centred symbol is inside the corresponding area when it is drawn with the specified offset from the centre of gravity. IHO S-64 contains examples of cases which an ECDIS at least shall be able to handle. |  |
| HP-04 |  | 8.5.4 |  | Technical | About fill patterns and textures We just want to inform that we support the drafted changes for fill patterns and textures for areas | No further change needed as the drafted change is good |  |
| HP-05 |  | 10.1.2 |  | General | About unknown and missing attributes This issue is severe. It seems that in practice so far IHO has not been able to force member states to follow IHO standards. It is known that IHO has an attempt to improve the situation by publishing a new edition of IHO S-58 Checks for ENC charts. This new edition is assumed to form a legal basis to declare that an ENC charts which do not pass S-58 is not a legal version to be published as an ENC to be used by an ECDIS mandated by IMO. The drafted change for 10.1.2 assume that even after new edition of S-58 the situation of non-legal ENC charts will continue. If this is the case then every look-up table entry and every CSP shall be amended to define what to do in case the mandatory attribute is missing and what to do when mandatory attribute has unknown value. It does not help anybody to say that the IHO is powerless in this case, if what to do for presentation is left open for multiple interpretations. | If the principle of IHO member states slipping from the IHO rules is accepted, then IHO shall review and, if required amend, each look-up table entry and each CSP to be fail-to-safe for two separate cases of missing or unknown mandatory attribute. |  |
| HP-06 |  | 10.1.3 |  | Technical | About Cell overlap Background: IHO inability to control member states and force them to follow what is specified in the S-57 Current proposal will create a new ECDIS anomaly. Reason is that current proposal will make users to expect that OEM will handle the difficult situation when IHO member state(s) has produces overlapping cells, but the proposed text does not give any instruction how to do that. The result will be as many styles as there are different OEMs. The serious overlapping problem is not only related to drawing but also related to chart based alerts and indications, which should be based on the largest scale available. Mixed drawing of the skin of the earth objects is very difficult. Reason is that they cannot be drawn as an overlay to each other. Mixed drawing of other objects than the skin of the earth is possible, but will lead to end user confusion with double contour, double soundings, double aids to navigation, etc. IHO DIPWG shall agree what to do in this case. Technical alternatives include at least - Fixed drawing priority based on a criteria, for example alphabetical order of name of the producer. This priority used also for alerts and indication. - Mixed drawing of non shin of the earth objects. Fixed priority of drawing of skin of the earth objects based on a criteria, for example alphabetical order of name of the producer. This alternative is against current drawing methods and will cause large software changes. Also it may be too difficult for some existing ECDIS to implement. - Propose your own better idea | Force IHO member states to follow what is specified in the S-57 If this is done the complete chapter about data overlap can be removed. If above not possible to accept, then IHO DIPWG must define exact drawing rules plus alerts and indications rule for overlapping cells. |  |
| HP-07 |  | 10.2 |  | Technical | About spatial queries to test correct operation The requirement is too loose. The target is understandable and we can support it, but the text is too open to allow multiple interpretations. We also think that requesting “mariner to test this” is a typographical mistake. Marine is the end user and inspection of this requirement shall be done both by the test person of the manufacturer and by the inspector of the type approval body. | Change as below To successfully use the Presentation Library ECDIS must be capable of performing spatial queries on ENC data during import and symbolisation. Spatial query is understood as possibility to inspect graphical location and numerical value of spatial coordinates associated with a charted object. Spatial query could be available as a part of cursor pick (see 10.8) or as an independent function. Due to the complex nature of these queries it is recommended that the mariner inspector of this requirement tests extensively that all required geometric primitives are accounted for in these tests and that the conditional symbology procedures are thoroughly understood during manufacture. Refer to section 12 for further details of which queries are required. IHO S-64 contains examples of cases which an ECDIS at least shall be able to handle. |  |
| HP-08 |  | 10.4.1 |  | Technical | About date dependency We agree the basic principle of this change – control of display by viewing date, but we have some issues for the proposed method. Mariners do not see ECDIS charts as individual chart cells but as a seamless database. Therefore a concept to set individual dates for a cell or a set of cells is very difficult to implement so that the mariner could intuitively understand how to operate the system. This is a usability issue. Our opinion is that there should be two mariner selectable dates “start viewing date” and “end viewing date”. Then all objects within these dates should be displayed. If both dates are equal then the result is like with a single selectable viewing date which the mariner can select and which is applied to the complete database within an ECDIS. The date range will simplify route planning as the mariner can see what will happen during the date range of his intended sailing. If the mariner want to see the clean situation before of after he can always set both dates either for the day of departure or for the day of arrival. If the viewing date is different from current date or the date range do not include current date, then there should a permanent indication for the mariner. The proposed warning text is absolutely too long to be permanently displayed. We must note that during route planning the mariner must daily use the concept of setting the viewing date and this warning would then permanently obscure a large part of the chart screen. Further we disagree the concept of “do not use for navigation”. Selecting a date in the future is part of the daily task of the navigator. We cannot declare that in such case he needs to refer to something else than his ECDIS. We prefer to request the mariner to “Note” that just now he is looking for a future situation. The DATSTA, DATEND, PERSTA, PEREND attributes do not include time, only date. Therefore selection should be by “date” instead of “data and time”. | Change as below To provide the Mariner with effective route planning capabilities and for the look-ahead function during route monitoring ECDIS must allow the display of date dependent chart data based on a selectable date range (start viewing date and end viewing date). During route planning and monitoring the mariner must be able to select a date or date range and time in the future to display all chart objects. Change as below: IMPORTANT: When the viewing date or date range do not include current date the mariner must be informed by a permanent warning on the chart display that the time and date have been adjusted using one of . Please use the alternative texts below; The information contained within the current chart display has been modified to display date dependent features, do not use for navigation. Note that the display is based on viewing date dd mmm yyyy. or Note that the display is based on viewing date range from dd mmm yyyy to dd mmm yyyy. Where dd is day, mmm is month and yyyy is year; for example 27 Mar 2013. |  |
| HP-09 |  | 10.5.7 |  | General | About shallow water pattern We agree with proposed change | No further change needed as the drafted change is good |  |
| HP-10 |  | 10.5.9 |  | Technical | About detection and indication of navigational hazards We agree basic principle of this change. Our opinion is that the method of indication shall be defined to avoid an open door for multiple different implementation by different OEMs. This feature shall not be subject to variations and shall be identical in every ECDIS. | Change as below The IMO Performance Standard for ECDIS requires that "ECDIS should give an indication, if, within a specified time set by the mariner, own ship pass closer than the user-specified distance for a danger (e.g. obstruction, wreck, rock) that is shallower than the mariner's safety contour or an aid to navigation”. The following table lists the S-57 objects and their attributes that satisfy this condition and must precipitate an indication within the ECDIS. The point, line or area shall be graphically indicated using the presentation named as “dnghlt” in the look-up tables. |  |
| HP-11 |  | 10.5.10 |  | Technical | About detection and indication of areas for which a special condition exist We agree basic principle of this change. Our opinion is that the method of indication shall be defined to avoid an open door for multiple different implementation by different OEMs. This feature shall not be subject to variations and shall be identical in every ECDIS. | Change as below The IMO Performance Standard for ECDIS requires that "ECDIS should give an alarm or indication, as selected by the mariner, if, within a specified time set by the mariner, own ship will cross the boundary of a prohibited area or area for which special conditions exist". The point, line or area shall be graphically indicated using the presentation named as “dnghlt” in the look-up tables. |  |
| HP-12 |  | 10.5.11 |  | General / Technical | About visualization of safety contour Title of this chapter is “Visualization of safety contour”. Content is just an isolated table entry without any text. This does not make any sense for us. We feel that drafting of this was not completed for the version to be reviewed. | Change content of the 10.5.11 as below The safety contour is defined as an edge between safe and unsafe skin of the earth objects. This edge shall be visualized using the presentation named as “safcon” in the look-up tables. Add following definition into the “Annex A_Appendix D Lookup lines.docx” "safcon","","LS(SOLD,3,DEPSC)","8","O","DISPLAYBASE","13000" |  |
| HP-13 |  | 10.5.12 |  | Technical | About detection of safety contour We agree basic principle of this change. The drafted change miss separate IMO requirement to only indicate instead of alarm when planning instead of monitoring. Further our opinion is that the method of indication shall be defined to avoid an open door multiple different implementation by different OEMs. This feature shall not be subject to variations and shall be identical in every ECDIS. | Change as below The IMO Performance Standard for ECDIS [2] requires that for route monitoring the "ECDIS should give an alarm if the ship, within a specified time set by the mariner, is going to cross the safety contour". The IMO Performance Standard for ECDIS requires that for route planning the "ECDIS should give an indication if the mariner plans a route across an own ship’s safety contour". The following table specifies the S-57 objects, conditions and geometry that constitute the safety contour and must therefore be used when raising the safety contour alarm or when indicating planning across the safety contour. The point, line or area shall be graphically indicated using the presentation named as “dnghlt” in the look-up tables. |  |
| HP-14 |  | 10.5.12 | Table | Technical | About detection of safety contour – 2nd detail We propose clarification for use of the UDWHAZ for the safety contour | Change 8th row and 2nd column of the table as below CSP UDWHAZ04 “DANGER=True” |  |
| HP-15 |  | 10.6.3 |  | Technical | About light description strings We can agree the principle to change from language C based code to textual description. However the textual description is not as precise as the C-code. Especially the test case available in the famous IHO Check Data Set is left open for interpretation. The issue is decimal numbers or no decimal numbers for numeric fields. | Add after the table “units of measure suffixes” as below Default for each numeric value: signal period, height and value of nominal range is no decimals. If the value of the attribute has non-zero decimal part then the value is displayed with one decimal number. |  |
| HP-16 |  | 10.8.4 |  | Technical | About sorting in the cursor pick report Our comment about sorting by significance is that this “significance” must be available for the ECDIS software without mandatory unclear hard-coding into the software itself. The best candidate for sorting is the drawing order given by the drawing priority. This information is available in the look-up tables and can be made machine readable either through the DAI-file or by internal method of the OEM. The drawing priority parameter for each object has been carefully set by the cartographers of the IHO and values have already matured in practical use of the S-52 standard for over 10 years. | Change as below A general cursor enquiry may shall be sorted by importance drawing priority of the object class using the IMO category or by the significance of the attribute, the most significant attributes being those used as defined in the look-up table for symbolizing |  |
| HP-17 |  | 10.9.1 |  | General | About UI or HMI colours IHO as an organization is not responsible for the UI (User Interface) or HMI (Human Machine Interface) colours. This area is clearly controlled by IMO MSC 191(79) and by corresponding IEC 62288. If IHO want to specify something then IHO can only request for clear separation between chart area and UI. Such separation could be for example at least 2 millimetre boundary consisting of a solid line of single colour followed by a neutral background colour used in the UI. | Remove this chapter which does not belong to IHO controlled domain. |  |
| HP-18 |  | 10.9.2 |  | General | About essential navigation information IHO as an organization is not responsible for the navigation related functionality of an equipment displaying chart content based on IHO S-52. IMO has defined functionality including mandatory displayed elements and indications for all typical system which shall or may display chart. For example IMO MSC.74(69) for track control, IMO MSC.192(79) for Radar, IMO MSC.232(82) for ECDIS and IMO MSC.252(83) for INS. | Remove this chapter which does not belong to IHO controlled domain. |  |
| HP-19 |  | 10.9.3 |  | Technical | About hover-over function A specification written as “are most suitable” will only cause more future anomalies. The IHO specification should not be open for multiple interpretations. | Change as below The hover over function must not be used for every charted object within the display, it is acknolwledged that following feature objects are only allowed most suitable for the application of the function. LIGHTS BUOYS BEACONS LANDMARKS |  |
| HP-20 |  | 14 | Whole chapter | General / Technical | About rewritten CSPs All technical descriptions are made in a new style. The version submitted for review seems to contain a huge number of already “not passing the review” indicating comments. Some picks from the document: - My opinion, it is better to remove …. - There was the following action: “show the selected symbol …..”. It should be changed ….” All above indicate that the rewritten chapter 14 is far away from being acceptable for publishing. We wonder how this kind of large task could be completed with a safe and sound result. The risk of major disaster after publishing of the new edition of the S-52 is very high. | Consider keeping this part unchanged. Consider only minor added notes to clarify the content of the previous edition 3.4. Consider keeping the “portrayal for S-100/S-101 project” as a separate task and not included in the S-52 ed 3.5 or 4.0. This would allow enough time and resources to a have both S-52 and S-100/S-101 as sound and solid standards. |  |
| HP-21 |  | 15.2 |  | Technical | About viewing group layers We support the idea to define clear names for the viewing groups that can be used in the ECDIS human machine interface (HMI). However we had difficulties to find this clarification. In the end we assumed that the table with title “EXAMPLE” might be the said clarification. This issue shall be documented in a more clear way which does not leave any door for interpretations or anomalies. In the reviewed draft there are viewing group layers defined both for chart and for mariner features. In the draft they are all “might be implemented”. Our opinion is that for the chart related viewing group layers there shall be mandatory implementation and the mariner related viewing groups shall not be defined by IHO as mandatory. Our opinion is that IMO in their IMO MSC.74(69) for track control, IMO MSC.192(79) for Radar, IMO MSC.232(82) for ECDIS and IMO MSC.252(83) for INS has defined the rules for the mariner objects. Our proposal is that the IHO S-52 document include only list of the mandatory chart related viewing group layers. Further our proposal is that the mandatory viewing group layers are seen as “at least” principle. Which means that an OEM is allowed to have more viewing group layers that documented in the “at least” list. Such additional viewing group layers shall be organized in the Human Machine Interface under the corresponding viewing group layer name documented in the “at least “ list. CONTINUE IN NEXT PAGE | Change the example as a new chapter and use more clear text. Replace EXAMPLE The following is an example of how the above viewing groups might be implemented in an ECDIS: As below 15.2.1 Mariner control of viewing group layers All individual view groups, for example 21010, 21020 etc., are not intended to be made available for mariner to select in the Human Machine Interface (HMI) of the ECDIS. For mariner selection the individual viewing groups are collected under viewing group layers. For standardization of the ECDIS HMI and for enabling generic training of the mariners at least following viewing group layers shall be implemented to control display on/off of charted objects. An ECDIS may provide more display on/off controls that available in this table, but in such case the viewing group layer names in this table shall be used as titles of such more detailed selections. An example of more detailed selections The table has viewing group layer “depth contour, currents and magnetics”. A manufacturer may subdivide this as “depth contours”, “depth contour labels”, “currents” and “magnetics” Change also column titles of the table as below Over-groupViewing group layer contentName of the viewing group layer in the ECDIS HMI Delete from the table the rows related to “Standard display – mariners features” and “Other mariner features” as they are not part of IHO domain of the presentation of the charted objects. See also annex of this comment form. |  |
| HP-21 continue |  |  |  |  | CONTINUATION FROM PREVIOUS PAGE “at least” list of viewing group layers Standard display - Unknown object - Chart data coverage - Land features - Area of depth less than safety contour - Water and seabed features - Traffic route - Cautionary areas - Boyos and beacons - Lights - Fog signals - Radar - Services (pilot, signal stns) Other - Information about chart data - Land features - Soundings - Depth contours, currents, magnetics - Seabed and obstructions - Services and small craft facilities |  |  |
| HP-22 |  | 15.3 |  | Technical | About text group layers The display control for text shall be similar as for viewing groups/viewing group layers. For text name “text group layer” shall be used instead of “viewing group layer”. There shall be similar “at least” mandatory to implement list of “text group layers” as for the “viewing group layers”. The list of “at least” mandatory “text group layers” shall include following - Important text - Other text As for viewing group layers an OEM is allowed to have more text group layers that documented in the “at least” list. Such additional text group layers shall be organized in the Human Machine Interface under the corresponding text group layer name documented in the “at least “ list. | Change as below. Text groupings are All text is organized for text groups. Individual text groups, for example 21, 23 etc., are not intended to be made available for mariner to select in the Human Machine Interface (HMI) of the ECDIS. For mariner selection the individual text groups are collected under text group layers. For standardization of the ECDIS HMI and for enabling generic training of the mariners at least following text group layers shall be implemented to control display on/off of the text of charted objects. An ECDIS may provide more display on/off controls that available in this table, but in such case the text group layer names in this table shall be used as titles of such more detailed selections. An example of more detailed selections The table has text group layer “Other text”. A manufacturer may subdivide this as “names”, “light description” and “other”. Change also the structure of the chapter similar as for viewing groups. See annex of this comment form. |  |

Annex: Worked out version for chapters 15.2, 15.2.1 and 15.3

Note: deletion are marked with blue strikeout (example) and additions are marked with red underline (example)

## 15.2		Viewing Group Layers

Numbering scheme for viewing groups (Mariners' information in italics)

| DISPLAY BASE | STANDARD DISPLAY | OTHER INFORMATION |
| --- | --- | --- |
| 00000-09999 reserved for administrative purposes |  |  |
| 10000 reserved 40000 reserved | 20000 reserved 50000 reserved | 30000 reserved 60000 reserved |
| 11000 A,B information about the chart display 41000 tools | 21000 A,B 51000 tool | 31000 A,B 61000 tools |
| 12000 C, D, E, F land features 42000 own ship, planned route | 22000 C, D, E, F 52000 own ship etc | 32000 C, D, E, F 62000 own ship etc |
| 13000 H, I depths & currents 43000 mariners' features | 23000 H,I 53000 mariners' features | 33000 H,I 63000 mariners' features |
| 14000 J,K,L obstructions, pipelines 44000 other vessels | 24000 J,K,L 54000 other vessels | 34000 J,K,L 64000 other vessels |
| 15000 M traffic,routes 45000 manufacturers' features | 25000 M 55000 manufacturers' features | 35000 M 65000 manufacturers’ features |
| 16000 N special areas 46000 mariners' assignments | 26000 N 56000 mariners' assignments | 36000 N 66000 mariners' assignments |
| 17000 P,Q,R,S buoys, beacons, lights, radar 47000 reserved for mariners' information | 27000 P,Q,R,S 57000 reserved | 37000 P,Q,R,S 67000 reserved |
| 18000 T,U services & small craft facilities 48000 reserved for mariners' information | 28000 T,U 58000 reserved | 38000 T,U 68000 reserved |
| 19000-19999 reserved 49000-49999 reserved | 29000-29999 reserved 59000-59999 reserved | 39000-39999 reserved 69000-69999 reserved |
| 70000-99999 reserved for future use. |  |  |

Note: 	These viewing groups reflect the display category, but they do not set it. Display Category is set by field 6 of the look-up table.

Gaps between sets and groups have been left deliberately to allow for future expansion. "na" means that a particular set or group is not yet assigned

CHART INFORMATION - DISPLAY BASE

| RESERVED |  |
| --- | --- |
| 10000-10999 | Reserved for chart information |
| A, B CHART FURNITURE |  |
| 11000 | Information about the Chart Display |
| 11010 11020 | cursor [symbol SY(CURSRA01)] na (not assigned) |
| 11030 | scalebar, latitude scale [SY(SCALEB10),SY( SCALEB11)] |
| 11040 | north arrow [SY(NORTHAR1)] |
| 11050 | no data [colour NODTA, AP(NODATA03)], unsurveyed (UNSARE), incompletely surveyed area |
| 11060 | Non-HO data boundary LC(NONHODAT) |
| C, D, E, F TOPOGRAPHY AND INFRASTRUCTURE |  |
| 12000 | Land area |
| 12010 | land area (LANDARE) |
| 12200 | Dangers above water |
| 12210 | bridge (BRIDGE), pylon (PYLONS), overhead cable (CBLOHD), conveyor (CONVYR), overhead pipeline (PIPOHD), offshore platform (OFSPLF) |
| 12400 | Shoreline |
| 12410 | coastline (COALNE), ice shelf, glacier (ICEARE), shoreline construction (SLCONS), tie-up wall, dolphin (MORFAC), gate (GATCON, pile (PILPNT), crib, wellhead, ice boom (OBSTRN), floating dock (FLODOC), hulk (HULKES), pontoon (PONTON), oilboom (OILBAR), log boom (LOGPON), flood barrage (DAMCON, CATDAM3) |
| 12420 | dock (DOCARE), lock (LOKBSN), canal (CANALS), river (RIVERS) |
| H, I HYDROGRAPHY |  |
| 13000 | Safety Contour |
| 13010 | safety contour (from conditional symbology procedure DEPCNT03) |
| 13020 | na |
| 13030 | depth area (DEPARE), dredged area (DRGARE), |
| J, K, L SUBSEA FEATURES |  |
| 14000 | Dangers under water |
| 14010 | isolated underwater dangers in water deeper than the displayed safety contour (rocks, wrecks, obstructions, mooring cables from conditional symbology procedure) |
| M TRACKS AND ROUTES |  |
| 15000-15999 | na (not assigned) |
| N SPECIAL AREAS |  |
| 16000-16999 | na (not assigned) |
| P, Q, R, S AIDS TO NAVIGATION |  |
| 17000-17999 | na (not assigned) |
| T, U SERVICES |  |
| 18000-18999 | na (not assigned) |
| RESERVED |  |
| 19000-19999 | Reserved for chart information |

The groupings are given to illustrate the contents of the Display Base. All objects of this category should be permanently retained on the ECDIS display.

CHART INFORMATION - STANDARD DISPLAY

| RESERVED |  |
| --- | --- |
| 20000 | Reserved for chart information |
| A, B INFORMATION ABOUT THE CHART DISPLAY |  |
| 21000 | Information about the Chart Display |
| 21010 | Unknown object (magenta question mark) |
| 21020 | Generic Object (NEWOBJ01) |
| 21030 | Chart scale boundary, overscale data [AP(OVERSCO1)] |
| 21040 | Na |
| 21050 | Na |
| 21060 | Place-holder for geographic names (LNDRGN, SEAARE) |
| C, D, E, F NATURAL & MAN-MADE FEATURES, PORT FEATURES |  |
| 22000 | Major Coastal Features |
| 22010 | Riverbank (RIVBNK), lake (LAKARE), lakeshore (LAKSHR), sloping ground (SLOGRD), slope top (SLOTOP), dyke (DYKCON), causeway (CAUSWY), dam (DAMCON), |
| 22200 | Conspicuous landmarks: |
| 22210 | Radar conspicuous object - (any object with attribute CONRAD 1) |
| 22220 | Visually conspicuous object (any object with attribute CONVIS 1) |
| 22230 | Na |
| 22240 | Built up area (BUAARE) |
| H, I DEPTHS, CURRENTS, etc. |  |
| 23000 | Depths |
| 23010 | Area of depth less than the safety contour (DIAMON01 pattern) |
| 23020 | Na |
| 23030 | Swept area (SWPARE) |
| J, K, L SEABED, OBSTRUCTIONS, PIPELINES |  |
| 24000 | Seabed dangers |
| 24010 | Mooring cables (MORFAC, CATMOR6), (CBLSUB, CATCBL6), tunnel on Seabed (TUNNEL, BURDEP=0), sandwaves (SNDWAV) |
| M TRAFFIC ROUTES |  |
| 25000 | Routes and Tracks |
| 25010 | Leading line, clearing line (NAVLNE), traffic lane (TSSLPT), deep water route (DWRTPT), traffic separation area (TSEZNE), traffic separation line (TSELNE), traffic roundabout (TSSRON), traffic crossing (TSSCRS), precautionary area (PRCARE), traffic separation scheme boundary (TSSBND), deep water route centre line (DWRTCL), two way route part (TWRTPT), inshore traffic zone (ISTZNE). |
| 25020 | Recommended track (RECTRC), recommended traffic lane (RCTLPT), recommended route centreline (RCRTCL) |
| 25030 | Ferry route (FERYRT) |
| 25040 | Radar line (RADLNE), limit of shore radar (RADRNG) |
| 25060 | Radio calling in point (RDOCAL) |

| N SPECIAL AREAS |  |
| --- | --- |
| 26000 | Restricted and Cautionary Areas |
| 26010 | Restricted area (RESARE) |
| 26020 | Na |
| 26030 | Na |
| 26040 | Ferry route area (FERYRT), submarine transit lane (SUBTLN), military practice area (MIPARE), sea plane landing area (SPLARE), offshore production area (OSPARE) |
| 26050 | Caution area (CTNARE), fairway (FAIRWY) |
| 26200 | Information Areas. Protected Areas |
| 26210 | Fishing ground (FSHGRD), marine farm (MARCUL), |
| 26220 | Anchorage area (ACHARE), anchor berth (ACHBRT), |
| 26230 | Pipeline area (PIPARE), cable area (CBLARE) |
| 26240 | Dumping ground (DMPGRD), |
| 26250 | Cargo transhipment (CTSARE), incineration (ICNARE) |
| 26260 | Archipelagic sea lane (ASLXIS, ARCSLN) |
| P, Q, R, S BUOYS & BEACONS, LIGHTS, FOG SIGNALS, RADAR |  |
| 27000 | Buoys, Beacons, Topmarks, Lights, Fog Signals |
| 27010 | Buoy (BOYxxx), light float (LITFLT), mooring buoy (MORFAC, CATMOR7) |
| 27011 | Light vessel (LITVES) |
| 27020 | Beacon (BCNxxx) |
| 27025 | Daymark (DAYMAR) |
| 27030 | Na |
| 27040 | Direction of buoyage IALA buoyage regions (M_NSYS) |
| 27050 | topmarks (TOPMAR) - for paper chart symbols |
| 27060 | Na |
| 27070 | light (LIGHTS), |
| 27080 | fog signal (FOGSIG), retro-reflector (RETRFL) |
| 27200 | Radar |
| 27210 | racon (RTPBCN) |
| 27220 | na |
| 27230 | radar reflector (RADRFL) |
| T, U SERVICES & SMALL CRAFT FACILITIES |  |
| 28000 | Services |
| 28010 | pilot boarding point (PILBOP) |
| 28020 | signal station, traffic (SISTAT), sig. stn. warning (SISTAW) |
| RESERVED |  |
| 29000 | reserved for chart information |

OTHER CHART INFORMATION

| RESERVED |  |
| --- | --- |
| 30000 | reserved for chart information |
| A, B INFORMATION ABOUT THE CHART DISPLAY |  |
| 31000 | Information about the Chart Display |
| 31010 | accuracy of data (MPCCY), survey reliability (M_SREL), survey source (M_SSOR) quality of data (M_QUAL) |
| 31011 | symbol LOWACC01, identifying low accuracy data, applied to the spatial object of point and area wrecks, rocks and obstructions and to point land areas |
| 31020 | nautical publication (M_NPUB) |
| 31030 | information from attributes INFORM, TXTDSC, PICREP |
| 31040 | data scale and coverage (M_CSCL, M_COVR) |
| 31050 | na |
| 31060 | na |
| 31070 | na |
| 31080 | magnetic variation (MAGVAR), local magnetic anomaly (LOCMAG) |
| C, D, E, F NATURAL & MAN-MADE FEATURES, PORT FEATURES |  |
| 32000 | Natural Features |
| 32010 | dunes , hills (SLOGRD), ridge, clifftop (SLOTOP), contours and elevation (LNDELV) |
| 32020 | na |
| 32030 | trees , vegetation, mangrove (VEGATN), marsh (LNDRGN) |
| 32040 | na |
| 32050 | river (RIVERS) or lake (LAKARE); also rapids (RAPIDS), waterfall (WATFAL) |
| 32060 | Na |
| 32070 | tideway (TIDWAY), saltpan (SLTPAN) |
| 32080 | na |
| 32200 | Shore Structures |
| 32210 | na |
| 32220 | any of the following not classified as CONVIS1 (conspicuous): landmark (LNDMRK), building (BUISGL), tank, silo, water tower (SILTNK)cairn (CAIRNS), wall (FNCLNE), fort (FORSTC) |
| 32230 | na |
| 32240 | airport (AIRARE), runway (RUNWAY) |
| 32250 | railway (RAILWY), road (ROADWY), tunnel (TUNNEL), control point (CTRPNT) |
| 32260 | na |
| 32270 | quarry, refinery, power station, tank farm, wind farm, factory, timber yard (PRDARE) |
| 32280 | na |
| 32400 | Port Features |
| 32410 | harbour type (HRBFAC), customs check point (CHKPNT) [note: "small craft facilities" (SMCFAC) is in group 38210] |
| 32420 | na |
| 32430 | distance mark (DISMAR) |
| 32440 | berthing facility (such as wharf) (BRTFAC), berth number (BERTHS), mooring facility (such as bollard) (MORFAC), , gate (such as lock gate) (GATCON) , dry dock (DRYDOC), crane (CRANES) |
| 32450 | na |
| 32460 | gridiron (GRIDRN), |
| H, I DEPTHS, CURRENTS ETC |  |
| 33000 | Depths, Currents, Tide rips, etc |
| 33010 | Soundings (SOUNDG) |
| 33020 | depth contours (DEPCNT) other than the safety contour, line depth area (DEPARE) |
| 33021 | label for the safety contour |
| 33022 | label for contours other than the safety contour |
| 33030 | na |
| 33040 | water turbulence (WATTUR) |
| 33050 | tidal information (T_HMON, T_NHM, T_TIMS) |
| 33060 | current and tidal stream information (CURENT, TS_FEB, TS_PAD, TS_PNH, TS_PRH, TS_TIS) |
| J, K, L SEABED, OBSTRUCTIONS, PIPELINES |  |
| 34000 | Seabed Information: rocks, wrecks & obstructions, pipes & cables |
| 34010 | nature of seabed (SBDARE) |
| 34020 | spring (SPRING), sea weed (WEDKLP) |
| 34030 | na |
| 34040 | fish haven (FSHHAV), fishing stakes, etc. (FSHFAC) |
| 34050 | rocks (UWTROC), wrecks (WRECKS), obstructions (OBSTRN), which are not a danger to own-ship's navigation (these are all Display Base if a danger to own-ship) |
| 34051 | non-dangerous rocks (UWTROC), wrecks (WRECKS) and obstructions (OBSTRN) which have a VALSOU attribute and are not a danger to own-ship’s navigation (these objects are all Display Base if a danger to own-ship) |
| 34060 | na |
| 34070 | submarine cable (CBLSUB), submarine pipeline (PIPSOL) |
| M TRAFFIC ROUTES |  |
| 35000 | Routes |
| 35010 | na |
| N SPECIAL AREAS |  |
| 36000 | Administrative Areas, (by cursor enquiry) |
| 36010 | continental shelf (COSARE), |
| 36020 | harbour area (HRBARE) free port area (FRPARE), customs zone (CUSZNE) |
| 36030 | na |
| 36040 | fishery zone (FSHZNE) |
| 36050 | contiguous zone (CONZNE), exclusive economic zone (EXEZNE), national territorial area (NATARE), territorial sea (TESARE), territorial sea baseline (STSLNE), administration area (ADMARE) |
| P, Q, R, S BUOYS & BEACONS, LIGHTS, FOG SIGNALS, RADAR |  |
| 37000-37999 | na |
| T, U SERVICES & SMALL CRAFT FACILITIES |  |
| 38000 | Services |
| 38010 | radar station (RADSTA), radio station (RDOSTA) |
| 38020 | na |
| 38030 | coastguard station (CGUSTA), rescue station (RSCSTA) |
| 38200 | Small craft facilities |
| 38210 | small craft facilities (SMCFAC) |
| RESERVED |  |
| 39000 | reserved for chart information |

MARINERS' INFORMATION - DISPLAY BASE

| RESERVED |  |
| --- | --- |
| 40000 | Reserved for mariners’ information |
| TOOLS |  |
| 41000-41999 | na |
| OWN-SHIP, PLANNED ROUTES, PAST TRACKS |  |
| 42000 | Own ship |
| 42010 | Own ship (ownship), symbol or scaled version, together with heading line, beam bearing line and course and speed vector |
| 42200 | Selected Planned Route |
| 42210 | legline (leglin, select 1), way points (waypnt, select 1) |
| 42220 | Course to make good for selected leglines |
| MARINERS' FEATURES |  |
| 43000 | na |
| OTHER VESSELS |  |
| 44000 | na |
| MANUFACTURERS' FEATURES |  |
| 45000-45999 | Manufacturers’ Features |
| MARINERS' ASSIGNMENTS TO DISPLAY BASE |  |
| 46000-46999 | Mariners' and manufacturers' objects assigned to Display Base by the mariner |
| RESERVED |  |
| 47000-49999 | reserved for mariners’ information |

MARINERS' INFORMATION - STANDARD DISPLAY

| RESERVED |  |
| --- | --- |
| 50000 | reserved for mariners’ information |
| TOOLS |  |
| 51000-51999 | na |
| OWN-SHIP, PLANNED ROUTES, PAST TRACKS |  |
| 52000 | Notations on Selected Planned Route |
| 52010 | wheel-over line (wholin), selected route |
| 52020 | course to make good on leglines (leglin, select 1) of selected route |
| 52030 | planned position (pinpos), distance to go, and other notations on selected planned route |
| 52200 | Alternate Planned Route |
| 52210 | leglin (leglin, select 2), waypoint (waypnt, select 2) of alternate planned route |
| 52220 | na |
| 52230 | wheel-over line (wholin), alternate route |
| 52240 | other notations, alternate route |
| 52400 | Past Track |
| 52410 | event (events) |
| 52420 | na |
| 52430 | primary past track (pastrk, catpst 1) |
| 52440 | notations on primary past track |
| 52450 | na |
| 52460 | secondary past track (pastrk catpst 2) |
| MARINERS' FEATURES |  |
| 53000 | Mariners' features |
| 53010 | danger highlight (dnghlt) |
| 53020 | clearing line (clrlin) |
| 53030 | mariners' information note (marnot catnot 1) |
| 53040 | mariners' cautionary note (marnot catnot 2) |
| 53050 | mariners' feature (marfea) |
| 53060 | na |
| 53070 | na |
| 53080 | tidal current observed (tidcur, catcur 2), tidal current predicted (tidcur, catcur 1) |
| OTHER VESSELS |  |
| 54000 | Other Ships |
| 54010 | other ships (vessels) from radar |
| 54020 | notations on other ships (acqsta) |
| 54030 | other ships from other sources or undefined sources |

| MANUFACTURERS' FEATURES |  |
| --- | --- |
| 55000 | Manufacturers’ Features |
| 55010 | manufacturers' feature (mnufea, catnot 1) |
| 55020 | manufacturers' feature (mnufea, catnot 2) |
| MARINERS' ASSIGNMENTS TO STANDARD DISPLAY |  |
| 56000-66999 | Mariners' and manufacturers' Objects Assigned to Standard Display by the Mariner |
| RESERVED |  |
| 57000-59999 | reserved for mariners’ information |

OTHER MARINERS' INFORMATION

| RESERVED |  |
| --- | --- |
| 60000 | reserved for mariners’ information |
| TOOLS |  |
| 61000 | Tools |
| 61010 | electronic bearing line (ebline), variable range marker (vrmark) |
| 61020 | na |
| 61030 | range rings (rngrng) |
| 61040 | cursor, style B (cursor, cursty2) |
| 61050 | cursor reference point (refpnt) |
| OWN-SHIP, PLANNED ROUTES, PAST TRACKS |  |
| 62000 | Position fixes |
| 62010 | position fix (positn) |
| 62020 | position line (poslin) |
| MARINERS' FEATURES |  |
| 63000-63999 | na |
| OTHER VESSELS |  |
| 64000-64999 | na |
| MANUFACTURERS' FEATURES |  |
| 65000-65999 | Manufacturers’ Features |
| MARINERS' ASSIGNMENTS TO OTHER INFORMATION |  |
| 66000-66999 | Mariners' and manufacturers' objects assigned to other information by the mariner |
| RESERVED |  |
| 67000-69999 | reserved for mariners’ information |

EXAMPLE

## 15.2.1	Mariner control of viewing group layers

The following is an example of how the above viewing groups might be implemented in an

ECDIS:

All individual view groups, for example 21010, 21020 etc., are not intended to be made available for mariner to select in the Human Machine Interface (HMI) of the ECDIS.

For mariner selection the individual viewing groups are collected under viewing group layers.

For standardization of the ECDIS HMI and for enabling generic training of the mariners at least following viewing group layers shall be implemented to control display on/off of charted objects.  An ECDIS may provide more display on/off controls that available in this table, but in such case the viewing group layer names in this table shall be used as titles of such more detailed selections.

An example of more detailed selections

The table has viewing group layer “depth contour, currents and magnetics”.  A manufacturer may subdivide this as “depth contours”, “depth contour labels”, “currents” and “magnetics”

| Over-group Viewing group layer |  | Content Name of viewing group layer in the ECDIS HMI |  |  | viewing groups included |
| --- | --- | --- | --- | --- | --- |
| 1 |  | Display Base |  |  | 10000 - 19999, 40000 - 49999 |
| Standard display - chart: |  |  |  |  |  |
| 2 |  | unknown object |  | 21010 |  |
| 3 |  | chart data coverage |  | 21020 - 21060 |  |
| 4 |  | land features |  | 22010 - 22260 |  |
| 5 |  | area of depth less than safety contour |  | 23010 |  |
| 6 |  | water and seabed features |  | 23020 - 24010 |  |
| 7 |  | traffic routes |  | 25010 - 25040 |  |
| 8 |  | cautionary areas |  | 26010 - 26050 |  |
| 9 |  | information areas |  | 26210 - 26270 |  |
| 10 |  | buoys & beacons |  | 27010 - 27050 |  |
| 11 |  | Lights |  | 27070 |  |
| 12 |  | fog signals |  | 27080 |  |
| 13 |  | radar |  | 27210 - 27230 |  |
| 14 |  | services (pilot, signal stns) |  | 28010 - 28020 |  |
| Standard display - mariners' features: |  |  |  |  |  |
| 15 | notations on planned route |  | 52010 - 52030 |  |  |
| 16 | alternate planned route |  | 52210 - 52240 |  |  |
| 17 | past track |  | 52410 - 52440 |  |  |
| 18 | secondary past track |  | 52460 |  |  |
| 19 | mariners' features |  | 53010 - 53080 |  |  |
| 20 | other vessels |  | 54010 - 54030 |  |  |
| 21 | manufacturers' features |  | 55010 - 55020 |  |  |
| 22 | mariners' assignments to std. display |  | 56000 - 56999 |  |  |
| Other chart information: |  |  |  |  |  |
| 2315 | information about chart data |  | 31010 - 31070 |  |  |
| 2416 | land features |  | 32010 - 32460 |  |  |
| 2517 | Soundings |  | 33010 |  |  |
| 2618 | depth contours, currents, magnetics |  | 33020 - 33060 & 31080 |  |  |
| 2719 | seabed and obstructions |  | 34010 - 34070 |  |  |
| 2820 | services and small craft facilities |  | 38010 - 38210 |  |  |
| Other mariners' information: |  |  |  |  |  |
| 29 | tools for chartwork |  | 61010 - 61050 |  |  |
| 30 | position fixes |  | 62010 - 62020 |  |  |
| 31 | other vessels, mariners' & mfrs' features (not assigned at present). |  |  |  |  |

## 15.3	Text groupings

The text groupings are:

All text is organized for text groups.  Individual text groups, for example 21, 23 etc., are not intended to be made available for mariner to select in the Human Machine Interface (HMI) of the ECDIS.

| Value of text group | Text group Description | S-57 Object and Attribute Acryonmsacronyms |
| --- | --- | --- |
| 00-10 | reserved for future assignment by IHO. |  |
| Important Text |  |  |
| 10 |  |  |
| 11 | vertical clearance of bridges | BRIDGE, VERCLR, VERCCL VERCOP, VERCSA |
|  | vertical clearance of overhead cable | CBLOHD, VERCLR |
|  | vertical clearance of overhead pipeline | PIPOHD, VERCLR |
|  | vertical clearance of conveyor | CONVYR, VERCLR |
|  | bearing of navline | NAVLNE, ORIENT |
|  | recommended route | RCRTCL |
|  | deep water route centreline line | DWRTCL |
|  | recommended track | RECTRC |
|  | name and communications channel of radio calling-in point | RDOCAL, OBJNAM, COMCHA |
| Other Text |  |  |
| 20 |  |  |
| 21 | name or number of buoys | BOYxxx, OBJNAM |
|  | name or number of beacons | BCNxxx, OBJNAM |
|  | name or number daymarks | DAYMAR, OBJNAM |
|  | name or number light vessel | LITVES, OBJNAM |
|  | name or number light float | LITFLT, OBJNAM |
|  | name or number offshore platform | OFSPLF, OBJNAM |
| 22 | na (not allocated) |  |
| 23 | light description string |  |
| 24 | additional notes on chart data | INFORM, TXTDSC |
| 25 | nature of seabed | SBDARE, NATSUR |
| 26 | geographic names | LNDRGN, OBJNAM |
|  |  | SEAARE, OBJNAM |
|  |  | ACHARE, CATACH |
| 27 | value of magnetic variation | MAGVAR, VALMAG |
|  | value of swept depth | SWPARE, DRVAL1 |
| 28 | height of islet or land feature | LNDARE,HEIGHT |
| 29 | berth number | BERTHS, OBJNAM |
|  |  | ACHBRT, OBJNAM |
| 30 | na |  |
| *31 | national language text | NOBJNM, NINFOM, NTXTDS |
| 32-49 | reserved for IHO |  |
| 50-69 | mariners' text, including planned speed etc. |  |
| 70-79 | manufacturer’s text |  |
| 80-99 | future requirements (AIS etc.) |  |

For mariner selection the individual text groups are collected under text group layers.

For standardization of the ECDIS HMI and for enabling generic training of the mariners at least following text group layers shall be implemented to control display on/off of the text of charted objects.  An ECDIS may provide more display on/off controls that available in this table, but in such case the text group layer names in this table shall be used as titles of such more detailed selections.

An example of more detailed selections

The table has text group layer “Other text”.  An OEM may subdivide this as “names”, “light description” and “other”.

| Text group layer | Name of text group layer in the ECDIS HMI |  | Text groups included |
| --- | --- | --- | --- |
| 1 | Important text | 10 - 19 |  |
| 2 | Other text | 20 - 31 |  |

NOTE: *National text is a supplementary option for ECDIS. If used, the style should be similar to that of the Presentation Library.