Modification of the Feature Catalogue Model

Motivation:

The motivation for the change in the feature catalogue model was to reflect the changes in the general feature model. In addition some cleanup has been done in the model to remove redundant information. The main change is that there are now two types of associations, feature and information association and that those can have attributes.

Background:

The GFM in S-100 Version 1.0.0 does not allow to associate information types by means of association types. In addition the feature association types are not allowed to carry thematic attributes. Both are required for the modeling of nautical publications. Since workarounds were not satisfying the situation, the GFM was modified. One side effect of these changes is that the model of the feature catalogue needs to be changed as well. These modifications are proposed in this document. In addition a new version of the feature catalogue schema (xsd) will be created.

There are two other small changes to the model. The first is the addition of a numeric code for items to reflect the ISO 8211 encapsulation where codes are integer numbers. The second is the change of an alias from a simple string to a structure containing the value and optional a context of the alias.

Furthermore a pattern constraint for item codes is introduced into the model. The motivation of this is that the codes are intended to use as tag names in XML encapsulations. They must comply with the XML name constraint.

The pattern allows only codes starting with a letter from the Latin alphabet (A-Z or a-z). All other characters must be either Latin alphabet characters or digits (0-9) or the underline character (_). No other characters are allowed including all kind of white spaces.

The numeric code is also constraint to the interval 0 < numericCode < 65535.

Proposed changes (brief):

The main changes are:

- Add a new class S100_FC_Item to carry the common properties of all feature catalogue items. Those were repeated in three classes of the old model. This is only a cleanup and does not semantically changes the model.

- The class S100_FC_PropertyType was removed from the model. Derived classes are now derived from S100_FC_Item. The reason for this is that the removed class does not add any information to the model.

- The classes S100_FC_InformationRole and S100_FC_AssociationRole were replaced by only one class S100_FC_Role due to the changes in the GFM. (There is only one role type in the GFM now)

- There is a super class for S100_FC_FeatureType and S100_FC_InformationType: S100_FC_ObjectType. The reason for this is the common property ‘informationBinding’.

- A new class S100_FC_InformationAssociation was added to the model due to the change of the GFM.

- The classes S100_FC_InformationAssociation and S100_FC_FeatureAssociation are subject of inheritance now (to be in line with the GFM).

- The attribute ‘permittedValues’ of S100_FC_AttributeBinding was changed into an association. This is not a semantically change of the model.

- Adding the attribute ‘numericCode’ to S100_FC_Item.

- Adding a class S100_FC_Alias with attributes ‘value’ and ‘context’.

- Changing the attribute ‘alias’ in S100_FC_Item and S100_FC_ListedValue to a role of type S100_FC_Alias.

- Adding constraints for the attributes ‘code‘ and ‘numericCode’ of class S100_FC_Item

Proposed changes (detailed):

Figure 1 - The modified feature catalogue model

Elements of the model:

| Role Name | Name | Description | Mult. | Type | Remarks |
| --- | --- | --- | --- | --- | --- |
| Class | S100_FC_FeatureCatalogue | a feature catalogue contains its identification and contact information, and definition of some number of feature types with other information necessary for those definitions | - | - |  |
| Attribute | name | name for this feature catalogue | 1 | CharacterString |  |
| Attribute | scope | subject domain(s) of feature types defined in this feature catalogue | 1 | CharacterString |  |
| Attribute | fieldOfApplication | description of kind(s) of use to which this feature catalogue may be put | 0..1 | CharacterString |  |
| Attribute | versionNumber | version number of this feature catalogue, which may include both a major version number or letter and a sequence of minor release numbers or letters, such as “3.2.4a.” The format of this attribute may differ between cataloguing authorities. | 1 | CharacterString |  |
| Attribute | versionDate | effective date of this feature catalogue | 1 | Date |  |
| Attribute | producer | name, address, country, and telecommunications address of person or organization having primary responsibility for the intellectual content of this feature catalogue | 1 | CI_ResponsibleParty |  |
| Role | item | list of items defined by this feature catalogue; items are feature types, information types, feature associations, information associations, attributes, and roles | 1..* | S100_FC_Item | aggregation |
| Role | definitionSource | list of sources of definitions of items, and listed values that are defined by this feature catalogue. Usually those sources are feature data dictionaries. | 0..* | FC_DefinitionSource | aggregation |
| Role | dataSetAttribute | Attribute bindings for the data set level to specify meta information for the data set | 0..* | S100_FC_AttributeBinding | aggregation |

Table 1 - S100_FC_FeatureCatalogue

| Role Name | Name | Description | Mult. | Type | Remarks |
| --- | --- | --- | --- | --- | --- |
| Class | FC_Definition_Source | class that specifies the source of a definition | - | - |  |
| Attribute | source | actual citation of the source, sufficient to identify the document and how to obtain it | 1 | CI_Citation |  |

Table 2 - FC_DefinitionSource

| Role Name | Name | Description | Mult. | Type | Remarks |
| --- | --- | --- | --- | --- | --- |
| Class | FC_DefinitionReference | class that links a data instance to the source of its definition | - | - |  |
| Attribute | sourceIdentifier | information to locate the definition in the source document. The format of this information is specific to the structure of the source document. | 1 | CharacterString |  |
| Role | definitionSource | the source of the definition | 1 | FC_DefinitionSource |  |

Table 3 - FC_DefinitionReference

| Role Name | Name | Description | Mult. | Type | Remarks |
| --- | --- | --- | --- | --- | --- |
| Class | S100_FC_Item | abstract base class that defines the common properties of all items in the feature catalogue; items are feature types, information types, feature associations, information associations, attributes and roles | - | - | abstract |
| Attribute | name | name of the item | 1 | CharacterString |  |
| Attribute | definition | definition of the named type in a natural language | 1 | CharacterString |  |
| Attribute | code | Code that uniquely identifies the named type within the feature catalogue. | 1 | CharacterString | Pattern: [A-Za-z][A-Za-z0-9_]* |
| Attribute | numericCode | A numeric code, used in the ISO 8211 encoding. Only required for products with such an encoding. | 0..1 | PositiveInteger | 0 < numericCode < 65535 |
| Attribute | remarks | further explanations about the item | 0..* | CharacterString |  |
| Role | alias | equivalent name(s) of this item | 0..* | S100_FC_Alias |  |
| Role | definitionReference | the link to the source of definition | 0..1 | FC_DefinitionReference |  |

Table 4 - S100_FC_Item

| Role Name | Name | Description | Mult. | Type | Remarks |
| --- | --- | --- | --- | --- | --- |
| Class | S100_FC_NamedType | abstract base class that defines the common properties for object types and associations | - | - | abstract; derived from S100_FC_Item |
| Attribute | isAbstract | Indicates if instances of this named type can exists in a geographic data set. Abstract types cannot be instantiated but serve as base classes for other (non-abstract) types. | 1 | Boolean |  |
| Role | attributeBinding | list of bindings to attributes which describe the characteristic of this named type | 0..* | S100_FC_AttributeBinding | aggregation |

Table 5 - S100_FC_NamedType

| Role Name | Name | Description | Mult. | Type | Remarks |
| --- | --- | --- | --- | --- | --- |
| Class | S100_FC_ObjectType | abstract base class that defines the common properties for feature types and information types | - | - | abstract; derived from S100_FC_NamedType |
| Role | informationBinding | list of bindings to information types that can be associated to this object type by means of an information association | 0..* | S100_FC_InformationBinding | aggregation |

Table 6 - S100_FC_ObjectType

| Role Name | Name | Description | Mult. | Type | Remarks |
| --- | --- | --- | --- | --- | --- |
| Class | S100_FC_InformationType | class that defines all properties of an information type | - | - | derived from S100_FC_ObjectType |
| Role | superType | Indicates the information type from which an information type is derived. The sub type will inherit all properties from its super type: name, definition and code will usually be overridden by the sub type, although new properties may be added to the sub type. | 0..1 | S100_FC_InformationType |  |
| Role | subType | Indicates the information types which are derived from an information type. | 0..* | S100_FC_InformationType |  |

Table 7 - S100_FC_InformationType

| Role Name | Name | Description | Mult. | Type | Remarks |
| --- | --- | --- | --- | --- | --- |
| Class | S100_FC_FeatureType | class that defines all properties of a feature type | - | - | derived from S100_FC_ObjectType |
| Attribute | featureUseType | the use type of this feature type | 1 | S100_FD_FeatureUseType |  |
| Attribute | permittedPrimitives | the combination of 0 or more spatial primitives permitted for feature type | 0..* | S100_FC_SpatialPrimitiveType |  |
| Role | featureBinding | list of bindings to feature types that can be related to this feature type by means of a feature association | 0..* | S100_FC_FeatureBinding | aggregation |
| Role | superType | Indicates the feature type from which a feature type is derived. The sub type will inherit all properties from its super type: name, definition and code will usually be overridden by the sub type, although new properties may be added to the sub type. | 0..1 | S100_FC_FeatureType |  |
| Role | subtype | Indicates the feature types which are derived from a feature type. | 0..* | S100_FC_FeatureType |  |

Table 8 - S100_FC_FeatureType

| Role Name | Name | Description | Mult. | Type | Remarks |
| --- | --- | --- | --- | --- | --- |
| Class | S100_FC_InformationAssociation | An information association describes the relationship between an object (feature or information type) and an information type. An information association is unidirectional and has only one role for the direction to the information type. | - | - | derived from S100_FC_NamedType |
| Role | role | the role of the association. | 1 | S100_FC_Role |  |
| Role | superType | Indicates the information association from which an information association is derived. The sub type will inherit all properties from its super type: name, definition and code will usually be overridden by the sub type, although new properties may be added to the sub type. | 0..1 | S100_FC_InformationAssociation |  |
| Role | subtype | Indicates the information associations which are derived from an information association. | 0..* | S100_FC_InformationAssociation |  |

Table 9 - S100_FC_InformationAssociation

| Role Name | Name | Description | Mult. | Type | Remarks |
| --- | --- | --- | --- | --- | --- |
| Class | S100_FC_FeatureAssociation | A feature association describes the relationship between two feature types. A feature association is bidirectional and has a separate role for each direction. | - | - | derived from S100_FC_NamedType |
| Role | role | the roles of the association | 2 | S100_FC_Role |  |
| Role | superType | Indicates the feature association from which a feature association is derived. The sub type will inherit all properties from its super type: name, definition and code will usually be overridden by the sub type, although new properties may be added to the sub type. | 0..1 | S100_FC_FeatureAssociation |  |
| Role | subtype | Indicates the feature associations which are derived from a feature association. | 0..* | S100_FC_FeatureAssociation |  |

Table 10- S100_FC_FeatureAssociation

| Role Name | Name | Description | Mult. | Type | Remarks |
| --- | --- | --- | --- | --- | --- |
| Class | S100_FC_Role | a role which can be used in a feature association or an information association | - | - | derived from S100_FC_Item |

Table 11 - S100_FC_Role

| Role Name | Name | Description | Mult. | Type | Remarks |
| --- | --- | --- | --- | --- | --- |
| Class | S100_FC_Attribute | Abstract base class for the two kinds of attributes: simple attributes and complex attributes. Attributes carry the characteristics of named types | - | - | abstract; derived from S100_FC_Item |

Table 12 - S100_FC_Attribute

| Role Name | Name | Description | Mult. | Type | Remarks |
| --- | --- | --- | --- | --- | --- |
| Class | S100_FC_SimpleAttribute | attribute that carries a value | - | - | derived from S100_FC_Attribute |
| Attribute | dataType | the data type of this attribute | 1 | S100_FD_AttributeDataType |  |
| Attribute | uom | unit of measure used for values of this feature attribute | 0..1 | S100_UnitOfMeasure |  |
| Attribute | quantitySpecification | the physical quantity | 0..1 | S100_FD_QuantitySpecification |  |
| Role | constraints | constraints which may apply to the attribute | 0..1 | S100_FD_AttributeConstraints | composition |
| Role | listedValues | set of listed values for an enumerated attribute domain | 0..* | S100_FC_ListedValue | composition; applies only if data type is enumeration |

Table 13 - S100_FC_SimpleAttribute

| Role Name | Name | Description | Mult. | Type | Remarks |
| --- | --- | --- | --- | --- | --- |
| Class | S100_FC_ComplexAttribute | a complex attribute consists of a list of sub-attributes which can be both simple or complex attributes | - | - | derived from S100_FC_Attribute |
| Role | subAttributeBinding | list of bindings to the sub-attributes | 1..* | S100_FC_AttributeBinding | aggregation |

Table 14 - S100_FC_ComplexAttribute

| Role Name | Name | Description | Mult. | Type | Remarks |
| --- | --- | --- | --- | --- | --- |
| Class | S100_FC_ListedValue | value of an enumerated attribute domain, including its codes and definition | - | - |  |
| Attribute | label | descriptive label that uniquely identifies one value in the attribute domain | 1 | CharacterString |  |
| Attribute | definition | definition of the listed value in a natural language | 1 | CharacterString |  |
| Attribute | code | Numeric code that uniquely identifies the listed value in the attribute domain. This code should be identical with the code in the feature concept dictionary, if any. | 1 | PositiveInteger |  |
| Attribute | remarks | further explanations about the listed value | 0..* | CharacterString |  |
| Role | alias | equivalent name(s) of this listed value | 0..* | S100_FC_Alias |  |
| Role | definitionReference | the link to the source of the definition | 0..1 | FC_DefinitionReference |  |

Table 15 - S100_FC_ListedValue

| Role Name | Name | Description | Mult. | Type | Remarks |
| --- | --- | --- | --- | --- | --- |
| Class | S100_FC_AttributeBinding | class that is used to describe the specifics of how an attribute is bound to a particular named type or a complex attribute | - | - |  |
| Attribute | multiplicity | multiplicity defining how many instances of the attribute can be part of the named type or complex attribute | 1 | S100_Multiplicity |  |
| Attribute | sequential | describes if the sequence of the attributes is meaningful or not | 1 | Boolean | Applies only to attributes which may occur more than once. |
| Role | permittedValues | permissible values of this attribute | 0..* | S100_FC_ListedValue | Applies only to attribute of data type enumeration. |
| Role | attribute | the attribute that is bound to the item or the complex attribute | 1 | S100_FC_Attribute |  |

Table 16 - S100_FC_AttributeBinding

| Role Name | Name | Description | Mult. | Type | Remarks |
| --- | --- | --- | --- | --- | --- |
| Class | S100_FC_InformationBinding | class describing the use of an information type by an object type | - | - |  |
| Attribute | multiplicity | defining how many instances of the target information type can be linked to one instance of the object type | 1 | S100_Multiplicity |  |
| Attribute | roleType | the nature of the association end | 1 | S100_FC_RoleType |  |
| Role | association | the association used for the binding; defining also the role | 1 | S100_FC_InformationAssociation |  |
| Role | informationType | the target information type | 1 | S100_FC_InformationType |  |

Table 17 - S100_FC_InformationBinding

| Role Name | Name | Description | Mult. | Type | Remarks |
| --- | --- | --- | --- | --- | --- |
| Class | S100_FC_FeatureBinding | class describing the relationship from one feature type to another feature type by means of a feature association | - | - |  |
| Attribute | multiplicity | multiplicity defining how many instances of the target feature type can be linked to one instance of the source feature type | 1 | S100_Multiplicity |  |
| Attribute | roleType | the nature of the association end | 1 | S100_FC_RoleType |  |
| Role | association | the association used for the binding | 1 | S100_FC_FeatureAssociation |  |
| Role | role | The role used for the binding. It must be part of the association used for the binding and defines the end of the association. | 1 | S100_FC_Role |  |
| Role | featureType | the target feature type | 1 | S100_FC_FeatureType |  |

Table 18 - S100_FC_FeatureBinding

| Role Name | Name | Description | Mult. | Type | Remarks |
| --- | --- | --- | --- | --- | --- |
| Class | S100_FC_Alias | class describing an alias, that is another name for an item | - | - |  |
| Attribute | value | The name of the alias | 1 | CharacterString |  |
| Attribute | context | The context in which the alias is used. | 0..1 | CharacterString |  |

Table 19 - S100_FC_Alias

| Role Name | Name | Description | Remarks |
| --- | --- | --- | --- |
| Enumeration | S100_FC_SpatialPrimitiveType | specifies spatial primitives permitted for use with a feature instance |  |
| Literal | point | point spatial primitive | GM_Point |
| Literal | pointSet | point set spatial primitive | GM_Pointset |
| Literal | curve | curve spatial primitive | GM_OrientableCurve |
| Literal | surface | surface spatial primitive | GM_Surface |
| Literal | coverage | coverage spatial primitive |  |

Table 20 - S100_FC_SpatialPrimitiveType

| Role Name | Name | Description | Remarks |
| --- | --- | --- | --- |
| Enumeration | S100_FC_RoleType | defines the nature of a role |  |
| Literal | association | An association is used to describe a relationship between two feature types that involves connections between their instances. |  |
| Literal | aggregation | An aggregation association is a relationship between two feature types, in which one of the feature types plays the role of a container and the other plays the role of a containee. |  |
| Literal | composition | A composition association is a strong aggregation. In a composition association, if a container object is deleted then all of its containee objects are deleted as well. With other words containee objects cannot exists without the container object. |  |

Table 21 - S100_FC_RoleType