S-100 – Part 4b

Metadata for Imagery and Gridded Data

Contents

4b-1	Scope	1

4b-2	Normative references	1

4b-3	Imagery and gridded data metadata	2

4b-3.1	Associated ISO standards	2

4b-3.2	Metadata packages	2

4b-3.2.1	Metadata Entity Set for Imagery	4

4b-3.2.2	Data quality information for Imagery	4

4b-3.2.3	Spatial representation information for Imagery	4

4b-3.2.4	Content information for Imagery	4

4b-3.2.5	Acquisition Information for Imagery	5

4b-4	UML diagrams and data dictionary	5

# Scope

The general scope of parts 4-A, 4-B and 4-C has been described at the beginning of part 4-A. This part concerns itself specifically with the growing requirement to manage large volumes of imagery and gridded data which most hydrographic organizations have in addition to handling the vector data. There are many different imagery and gridded data formats and these types of datasets are often stored on distributed systems leading to problems of data discovery, management and exchange.

The production of imagery and gridded data follows the processes that usually begin with the collection of data, scanning of charts and reference documents, and other sensing methods. These types of datasets are often used for the production of paper charts, Electronic Navigational Charts (ENCs), Raster Navigational Charts, and nautical publications. Their production processes need to be documented in order to maintain quality control over the end products. Furthermore, metadata about the geometry of the measuring process and the properties of the measuring equipment needs to be retained with the raw data in order to support the production and maintenance processes.

ISO 19115 defines the guidelines for describing geographic information and services. Although its model does make some provision for imagery and gridded data, these requirements were not fully developed at the time of initial publication in 2003. This metadata part of S-100 is based on ISO 19115 Part 2:2008 which was produced to provide the additional structure to more extensively describe the derivation of geographic imagery and gridded data, and it is intended to augment ISO 19115.

# Normative references

The following referenced documents are indispensable for the application of this document. For dated references, only the edition cited applies. For undated references, the latest edition of the referenced document (including any amendments) applies.

ISO/TS 19103, Geographic information — Conceptual schema language

ISO 19107:2003, Geographic information — Spatial schema

ISO 19115:2005, Geographic information — Metadata

ISO 19115-2:2008 Geographic information – Metadata – Part 2: Extensions for imagery and gridded data

ISO/TS 19139 Geographic information — Metadata — XML schema implementation

ISO 19119:2005 Geographic information -- Services

IHO S-61  Product Specification for Raster Navigational Charts

# Imagery and gridded data metadata

ISO 19115 identifies the metadata required to describe digital geographic data, and the extensions described in this section identify the metadata required to describe digital geospatial imagery and gridded data. Digital geospatial imagery and gridded metadata may also be provided for aggregations of datasets.

## Associated ISO standards

ISO 19115 is designed to be the general metadata standard applicable to all geographic data sets. It identifies a set of core metadata derived from the many metadata elements and also specifies the conditions under which they should be used (i.e. mandatory, conditional, or optional). Although there is some service metadata in ISO 19115, (particularly in the area of identification), much of the service metadata is defined in ISO 19119 (Services). ISO 19115 makes provision for limited metadata describing spatial and temporal schemas.

ISO 19115 - Part 2 extends the metadata defined in ISO 19115 and identifies additional metadata (such as data quality, spatial representation, content, and acquisition information), required to describe imagery and gridded data.  It provides information about the properties of the measuring equipment used to acquire data, geometry of the measuring processes employed by the equipment, and production processes used to digitize the raw data.

Geolocation information is a very important metadata component required for imagery. ISO 19115 and 19115 - Part 2 may not include sufficient geolocation metadata for imagery and gridded data. It may therefore be necessary to reference ISO 19130. This standard specifies additional information required to support geolocation and also defines how sensor measurements and geolocation information are logically associated. The georeferencing information in ISO 19130 is a subset of that described in ISO 19115 Part 2.   In order to develop a full set of imagery metadata, it may be necessary to combine the relevant sections from ISO 19115 and 19115 - Part 2, with the geolocation information or sensor properties from ISO 19130.

ISO 19139 - XML schema implementation, expands ISO 19115 by defining new constraint types that further refine the metadata elements for implementation. It also defines the rules used for deriving an XML schema from the ISO abstract UML models.

## Metadata packages

The relationships between the packages contained in ISO 19115 and the extensions for geospatial imagery and gridded data are illustrated in Figure 4b-1 below. These metadata extensions have been fully documented using both UML models and a data dictionary, in ISO/TC211 19115 – Part 2 - Annex A and Annex B respectively.

Figure 4b-1 — Metadata packages

It should also be noted that, to ensure global uniqueness, ISO/TS 19103 requires that all class names must be defined by a bi-alpha prefix that identifies the package to which a class belongs. ISO 19115 uses the prefixes MD (Metadata), CI (Citation), DQ (Data quality), EX (Extent), and LI (Lineage). To differentiate between entities used in ISO 19115 and those used in ISO 19115 Part 2 (Extensions for imagery and gridded data), the MI prefix is uses for imagery and gridded metadata, and LE and QE are used for extended Lineage and Data quality entities respectively. Table 1 contains the list of package identifiers for the classes used for metadata.

Table 4b-1 —UML Package Identifiers

| Identifier | Information Type | Standard |
| --- | --- | --- |
| MD | Metadata | ISO 19115 |
| MI | Metadata for Imagery | ISO 19115-2 |
| DQ | Data Quality | ISO 19115 |
| QE | Data quality Extended | ISO 19115-2 |
| CI | Citation | ISO 19115 |
| LI | Lineage | ISO 19115 |
| LE | Lineage Extended for Imagery | ISO 19115-2 |
| EX | Extent | ISO 19115 |
| GM | Geometry | ISO 19107 |
| MX | Metadata – XML schema | ISO/TS 19139 |

### Metadata Entity Set for Imagery

MI_Metadata is a subclass of MD_Metadata which aggregates the optional entity MI_AcquisitionInformation. See sections A.2.1 and B.2.1 of 19115 Part 2 for additional descriptive information and the data dictionary respectively.

### Data quality information for Imagery

Information about the sources and production processes used in producing an imagery or gridded dataset has been included in an additional Data Quality for Imagery package, as 19115 only makes provision for a general assessment of the quality.  The following additional classes are listed below. A more detailed description of the classes and associated data dictionary are provided in 19115-Part 2, sections A.2.2 and B.2.2 respectively).

- QE_CoverageResult is a specified subclass of DQ_Result and aggregates information required to report data quality for a coverage. It is based on concepts from 19115 and 19139.

- QE_Usability is a specified subclass of DQ_Element. It is intended to provide user specific

- quality information about a dataset’s suitability for a particular application;

- LE_ProcessStep is a specified subclass of LI_ProcessStep and contains additional information on the history of the algorithms used and processing performed to produce the data. LE_ProcessStep aggregates the following entities;

  - LE_Processing, describes the procedure (such as software used, parameters, and processing documentation) by which the algorithm was applied to generate the data from the source data. LE_Processing aggregates LE_Algorithm, which describes the methodology used to derive the data from the source data;

  - LE_ProcessStepReport identifies external information describing the processing of the data;

  - LE_Source is a specified subclass of LI_Source and describes the output of a process step.

### Spatial representation information for Imagery

This package contains information concerning the mechanisms used to represent spatial information. MI_GeoreferencingDescription contains addition information used to support georectification of the data and is an aggregation of the following entities:

- MI_Georectified contains check point information to further specify georectification details of the imagery or gridded data.  It aggregates MI_GCP;

- MI_Georeferenceable makes provision for the inclusion of additional information that can be used to geolocate the data. It aggregates MI_GeolocationInformation.

### Content information for Imagery

Although this package is part of 19115, the following entities have been included to better cater for imagery and gridded data;

- MI_Band (subclass of MD_Band) - defines additional attributes for specifying properties of individual wavelength bands in an imagery and gridded dataset;

- MI_ImageDescription (subclass of MD_ImageDescription) used to aggregate MI_RangeElementDescription;

- MI_CoverageDescription (subclass of MD_CoverageDescription) used to aggregate MI_RangeElementDescription;

- MI_RangeElementDescription - used to provide range elements used in a coverage dataset.

### Acquisition Information for Imagery

MI_AcquisitionInformation is an aggregate of the following entities:

- MI_Instrument (the measuring instruments used to acquire the data);

- MI_Operation,(the overall data gathering program to which the data contribute);

- MI_Platform (the platform from which the data were taken);

- MI_Objective,(the characteristics and geometry of the intended object to be observed);

- MI_Requirement (the user requirements used to derive the acquisition plan);

- MI_Plan,(the acquisition plan that was implemented to acquire the data).

Two additional classes are required to provide information on the acquisition of the data. These are:

- MI_Event, describes a significant event that occurred during data acquisition. An event can be associated with an operation, objective, or platform pass, and

- MI_PlatformPass, identifies a particular pass made by the platform during data acquisition. A platform pass is used to provide supporting identifying information for an event and for data acquisition of a particular objective.

# UML diagrams and data dictionary

The metadata schemas for the imagery and gridded data are included in ISO 19115 – Part 2 (Annex A) in the form of UML class diagrams.  These diagrams augment the UML diagrams shown in ISO 19115.

ISO 19115 – Part 2, Annex B contains the element and entity definitions for the metadata schemas defined in Annex A. The dictionary, in conjunction with the diagrams presented in Annex A and in combination with the UML diagrams and data dictionary presented in ISO 19115, serves to fully define the total abstract model for metadata.

Enumerations and their values provided in ISO 19115 are normative. User extensions to enumerations shall follow the rules as described in ISO 19115 and Annex C.